| 7 | VSS | S | Negative Supply Voltage (GND) |
| --- | --- | --- | --- |
| 8 | PDIO | DI_PD | OTP Programming Input and Data Input for Daisy Chain mode. Internal pull-down resistor (~74k). Connect to VSS if not used |
| 9 | DO | DO_T | Data Output of Synchronous Serial Interface |
| 10 | CLK | DI, ST | Clock Input of Synchronous Serial Interface; Schmitt-Trigger input |
| 11 | CSn | DI_PU, ST | Chip Select, active low; Schmitt-Trigger input, internal pull-up resistor (~50k) |
| 12 | PWM | DO | Pulse Width Modulation of approx. 244Hz; 1µs/step (opt. 122Hz; 2µs/step) |
| 13 | NC | - | Must be left unconnected |
| 14 | NC | - | Must be left unconnected |
| 15 | VDD3V3 | S | 3V-Regulator Output, internally regulated from VDD5V. Connect to VDD5V for 3V supply voltage. Do not load externally. |
| 16 | VDD5V | S | Positive Supply Voltage, 3.0 to 5.5 V |

| Pin | Symbol | Type | Description |
| --- | --- | --- | --- |
| 1 | MagINCn | DO_OD | Magnet Field Magnitude INCrease; active low, indicates a distance reduction between the magnet and the device surface. See Table 5 |
| 2 | MagDECn | DO_OD | Magnet Field Magnitude DECrease; active low, indicates a distance increase between the device and the magnet. See Table 5 |
| 3 | DTest1_A | DO | Test output in default mode |
| 4 | DTest2_B | DO | Test output in default mode |
| 5 | NC | - | Must be left unconnected |
| 6 | Mode_Index | DIO_PD | Select between slow (open, low: VSS) and fast (high) mode. Internal pull-down resistor. |

| Pin | Symbol | Type | Description |
| --- | --- | --- | --- |
| 1 | MagINCn | DO_OD | Magnet Field Magnitude INCrease; active low, indicates a distance reduction between the magnet and the device surface. See Table 5 |
| 2 | MagDECn | DO_OD | Magnet Field Magnitude DECrease; active low, indicates a distance increase between the device and the magnet. See Table 5 |
| 3 | DTest1_A | DO | Test output in default mode |
| 4 | DTest2_B | DO | Test output in default mode |
| 5 | NC | - | Must be left unconnected |
| 6 | Mode_Index | DIO_PD | Select between slow (open, low: VSS) and fast (high) mode. Internal pull-down resistor. |

| Parameter | slow mode (Mode = low or open) | fast mode (Mode = high, VDD5V) |
| --- | --- | --- |
| sampling rate | 2.61 kHz (384 µs) | 10.42 kHz (96µs) |
| transition noise (1 sigma) | 0.03° rms | 0.06° rms |
| output delay | 384µs | 96µs |
| max. speed @ 4096 samples/rev. max. speed @ 1024 samples/rev. max. speed @ 256 samples/rev. max. speed @ 64 samples/rev. | 38 rpm 153 rpm 610 rpm 2441 rpm | 153 rpm 610 rpm 2441 rpm 9766 rpm |

| OCF | COF | LIN | Mag INC | Mag DEC | Parity |
| --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 0 | even checksum of bits 1:15 |
|  |  |  | 0 | 1 |  |
|  |  |  | 1 | 0 |  |
|  |  |  | 1*) | 1*) |  |

| Status bits |  | Hardware pins |  | OTP: Mag CompEn = 0 (default) |
| --- | --- | --- | --- | --- |
| Mag INC | Mag DEC | Mag INCn | Mag DECn | Description |
| 0 | 0 | Off | Off | No distance change Magnetic input field OK (in range, ~45…75mT) |
| 0 | 1 | Off | On | Distance increase; pull-function. This state is dynamic and only active while the magnet is moving away from the chip. |
| 1 | 0 | On | Off | Distance decrease; push- function. This state is dynamic and only active while the magnet is moving towards the chip. |
| 1 | 1 | On | On | Magnetic input field invalid – out of recommended range: too large, too small (missing magnet) |

| Status bits |  |  | Hardware pins |  | OTP: Mag CompEn = 1 (red-yellow-green programming option) |
| --- | --- | --- | --- | --- | --- |
| Mag INC | Mag DEC | LIN | Mag INCn | Mag DECn | Description |
| 0 | 0 | 0 | Off | Off | No distance change Magnetic input field OK ( GREEN range, ~45…75mT) |
| 1 | 1 | 0 | On | Off | YELLOW range: magnetic field is ~ 25…45mT or ~75…135mT. The AS5145 may still be operated in this range, but with slightly reduced accuracy. |
| 1 | 1 | 1 | On | On | RED range: magnetic field is ~<25mT or >~135mT. It is still possible to operate the AS5145 in the red range, but not recommended. |
| All other combinations |  |  | n/a | n/a | Not available |

| Mode | Description | OutputMd1 | OutputMd0 |
| --- | --- | --- | --- |
| Default mode | Default AS5145 function DTEST1_A and DTEST2_B are not used the Mode_Index pin is used for selection of the decimation rate (low speed/high speed). | 0 | 0 |
| 10 bit incremental mode (low DNL) | DTEST1_A and DTEST2_B are used as A and B signal. In this mode the Mode_Index Pin is switched from input to output and will be the Index Pin. The decimation rate is set to 64 (fast mode) and cannot be changed from external. | 0 | 1 |
| 12 bit incremental Mode (high DNL) |  | 1 | 0 |
| Sync Mode | In this mode a control signal is switched to DTEST1_A and DTEST2_B. | 1 | 1 |

With the OTP bits OutputMd0 and OutputMd1 a specific mode can be selected.

| Resolution / bit | DTest1_A and DTest2_B pulses | Index Width |
| --- | --- | --- |
| 10 | 256 | 1 / 3 LSB |
| 12 | 1024 |  |

| Hystereses (LSB) | FS1 | FS0 | NOTE |
| --- | --- | --- | --- |
| 2 | 0 | 0 | default |
| 4 | 0 | 1 |  |
| 6 | 1 | 0 |  |
| 8 | 1 | 1 |  |

Table 7: Setup of Hysterese