CY8C41XX

PSOCTM Automotive 4100S

Based on Arm® Cortex®-M0+ CPU
Functional description

PSOCTM Automotive 4 is a scalable and reconfigurable platform architecture for a family of programmable embedded system controllers with an Arm® Cortex®-M0+ CPU, while being AEC-Q100 compliant. It combines programmable and reconfigurable analog and digital blocks with flexible automatic routing. The PSOCTM Automotive 4100S product family is a member of the PSOCTM Automotive 4 platform architecture. It is a combination of a microcontroller with standard communication and timing peripherals, a capacitive touch-sensing system (CAPSENSETM) with best-in-class performance, programmable general-purpose continuous-time and switched-capacitor analog blocks, and programmable connectivity. PSOCTM Automotive 4100S products will be upward compatible with members of the PSOCTM 4 platform for new applications and design needs.

Features

- Automotive Electronics Council (AEC) AEC-Q100 Qualified

- 32-bit Microcontroller subsystem

- 48 MHz Arm® Cortex®-M0+ CPU

- Up to 64 KB of flash with read accelerator

- Up to 8 KB of SRAM

- Programmable analog

- Two opamps with reconfigurable high-drive external and high-bandwidth internal drive and Comparator

modes and ADC input buffering capability. Opamps can operate in Deep Sleep low-power mode.

- 12-bit 1-Msps SAR ADC with differential and single-ended modes, and channel sequencer with signal averaging

- Single-slope 10-bit ADC function provided by a capacitance sensing block

- Two current DACs (IDACs) for general-purpose or capacitive sensing applications on any pin

- Two low-power comparators that operate in Deep Sleep low-power mode

- Programmable digital

- Programmable logic blocks allowing Boolean operations to be performed on port inputs and outputs

- Low-power 1.71 V to 5.5 V operation

- Deep Sleep mode with operational analog and 2.5 µA digital system current

- Capacitive sensing

- Capacitive sigma-delta (CSD) provides best-in-class signal-to-noise ratio (SNR) (> 5:1) and water tolerance

- Infineon-supplied software component makes capacitive sensing design easy

- Automatic hardware tuning (smart sensing algorithm)

- LCD drive capability

- LCD segment drive capability on GPIOs

- Serial communication

- Three independent run-time reconfigurable serial communication blocks (SCBs) with re-configurable I2C, SPI,

UART, or LIN slave functionality

- Timing and pulse-width modulation

- Five 16-bit timer/counter/pulse-width modulator (TCPWM) blocks

- Center-aligned, edge, and pseudo-random modes

- Comparator-based triggering of Kill signals for motor drive and other high-reliability digital logic applications

- Programmable GPIO pins

- 40-pin QFN, and 28-pin SSOP packages

- Any GPIO pin can be CAPSENSETM, analog, or digital

Datasheet	Please read the Important Notice and Warnings at the end of this document	002-15106 Rev. *L

www.infineon.com	page 1	2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Features

- Drive modes, strengths, and slew rates are programmable

- ModusToolboxTM design environment

- Integrated Development Environment (IDE) provides configuration, program development, and build with

debugging

- Applications Programming Interface (API) for all fixed-function and programmable peripherals using

Peripheral Driver Libraries (PDL)

- Temperature ranges

- A-grade: –40°C to +85°C

- S-grade: –40°C to +105°C

- E-grade: –40°C to +125°C[1]

Note

1. This device can also operate at temperatures exceeding 125°C (the high temperature of the AEC-Q100 Grade 1 operating range) for a limited amount of time depending on the mission profile of the application. Infineon provides a retention calculator to help estimate the retention lifetime based on the customers’ individual temperature profiles for operation throughout the –40°C to +150°C ambient temperature range. Go to the Infineon community page for more details.

Datasheet	2	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Table of contents

Table of contents

Functional description	1

Features	1

Table of contents	3

Logic block diagram 	5

1 Functional overview	6

1.1 CPU and memory subsystem	6

1.1.1 CPU	6

1.1.2 Flash 	6

1.1.3 SRAM	6

1.1.4 SROM	6

1.2 System resources	7

1.2.1 Power system	7

1.2.2 Clock system	7

1.2.3 IMO clock source	7

1.2.4 ILO clock source	7

1.2.5 Watch crystal oscillator (WCO)	8

1.2.6 Watchdog timer	8

1.2.7 Reset	8

1.3 Analog blocks	8

1.3.1 12-bit successive approximation register (SAR) ADC	8

1.3.2 Two opamps (Continuous-time block; CTB)	9

1.3.3 Low-power comparators (LPC)	9

1.3.4 Current DACs	9

1.3.5 Analog multiplexed buses	9

1.3.6 Programmable digital blocks	9

1.4 Fixed function digital	9

1.4.1 Timer/counter/PWM (TCPWM) block	9

1.4.2 Serial communication block (SCB)	9

1.5 LIN slave mode	10

1.6 GPIO	10

1.7 Special function peripherals	11

1.7.1 CAPSENSETM	11

1.7.2 LCD segment drive	11

2 Pinouts for 32-bit PSOCTM Automotive 4100S microcontrollers	12

2.1 Alternate pin functions	15

2.2 Function description	17

3 Power	19

3.1 Mode 1: 1.8 V to 5.5 V external supply	19

3.2 Mode 2: 1.8 V ± 5% external supply	19

4 Development support	21

4.1 Documentation	21

4.2 Online	21

4.3 Tools	21

5 Electrical specifications	22

5.1 Absolute maximum ratings	22

5.2 Device-level specifications	23

5.2.1 DC specifications	23

5.2.2 AC specifications	24

5.2.3 GPIO	24

5.2.4 XRES	25

5.3 Analog peripherals	27

Datasheet	3	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Table of contents

5.3.1 CTBm opamp	27

5.3.2 Comparator	33

5.3.3 Temperature sensor	34

5.3.4 SAR ADC	34

5.3.5 CSD and IDAC	36

5.3.6 10-bit CAPSENSETM ADC	39

5.4 Digital peripherals	40

5.4.1 Timer counter pulse-width modulator (TCPWM)	40

5.4.2 I2C 	41

5.4.3 SPI	41

5.4.4 UART	42

5.4.5 LCD	42

5.5 Memory	43

5.5.1 Flash 	43

5.6 System resources	44

5.6.1 Power-on reset (POR)	44

5.6.2 SWD interface	44

5.6.3 Internal main oscillator (IMO)	45

5.6.4 Internal low-speed oscillator (ILO)	45

5.6.5 Watch crystal oscillator	46

5.6.6 External clock	46

5.6.7 Block	46

5.6.8 PRGIO pass-through time	46

6 Ordering information	47

7 Packaging information for PSOCTM Automotive 4100S	52

7.1 Package diagrams	53

8 Acronyms	55

9 Document conventions	58

9.1 Units of measure	58

Revision history	59

Datasheet	4	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Logic block diagram

Logic block diagram

PSOCTM Automotive 4100S Architecture

System Resources Lite

TestMode Entry

POR

Reset Control

Clock Control

Sleep Control

ILO

Active/Sleep

Analog DFT

Digital DFT

Power Modes

DeepSleep

PWRSYS

AHB - Lite

Power

WIC

32-bit

Clock

Reset

XRES

WDT

Test

IMO

REF

I/O Subsystem

IOSS GPIO (5x ports)

CPU Subsystem

Peripherals

NVIC , IRQMUX

PCLK

SWD / TC

Cortex® M0+ 48 MHz

FAST MUL

SARMUX

Programmable Analog

System Interconnect ( Single Layer AHB )

SAR ADC (12-bit)

CTBm

2 x Opamp

High Speed I /O Matrix & 2 x Programmable I /O

Peripheral Interconnect ( MMIO )

Read Accelerator

FLASH64 KBK

SPCIF

34x GPIOs, LCD

5x TCP W M

SRAM Controller

SRAM 4 KB

CAPS ENS E

3x SCB-I2C/SPI/UART

2x P Comparator

ROM Controller

ROM 8 KB

WCO

Figure 1	Logic block diagram

Datasheet	5	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Functional overview

1	Functional overview

PSOCTM Automotive 4100S devices include extensive support for programming, testing, debugging, and tracing both hardware and firmware.

The Arm® SWD interface supports all programming and debug features of the device.

Complete debug-on-chip functionality enables full-device debugging in the final system using the standard production device. It does not require special interfaces, debugging pods, simulators, or emulators. Only the standard programming connections are required to fully support debug.

The PSOCTM Creator IDE provides fully integrated programming and debug support for the PSOCTM Automotive 4100S devices. The SWD interface is fully compatible with industry-standard third-party tools. The

PSOCTM Automotive 4100S family provides a level of security not possible with multi-chip application solutions or with microcontrollers.

It has the following advantages:

- Allows disabling of debug features

- Robust flash protection

- Allows customer-proprietary functionality to be implemented in on-chip programmable blocks

The debug circuits are enabled by default and can be disabled in firmware. If they are not enabled, the only way to re-enable them is to erase the entire device, clear flash protection, and reprogram the device with new firmware that enables debugging. Thus firmware control of debugging cannot be over-ridden without erasing the firmware thus providing security.

Additionally, all device interfaces can be permanently disabled (device security) for applications concerned about phishing attacks due to a maliciously reprogrammed device or attempts to defeat security by starting and interrupting flash programming sequences. All programming, debug, and test interfaces are disabled when maximum device security is enabled. Therefore, PSOCTM Automotive 4100S, with device security enabled, may not be returned for failure analysis. This is a trade-off that the PSOCTM Automotive 4100S allows the customer to make.

1.1	CPU and memory subsystem

1.1.1	CPU

The Cortex®-M0+ CPU in the PSOCTM Automotive 4100S is part of the 32-bit microcontroller subsystem, which is optimized for low-power operation with extensive clock gating. Most instructions are 16 bits in length and the CPU executes a subset of the Thumb-2 instruction set. It includes a nested vectored interrupt controller (NVIC) block with eight interrupt inputs and also includes a wakeup interrupt controller (WIC). The WIC can wake the processor from Deep Sleep mode, allowing power to be switched off to the main processor when the chip is in Deep Sleep mode.

The CPU also includes a debug interface, the SWD interface, which is a two-wire form of JTAG. The debug

configuration used for PSOCTM Automotive 4100S has four breakpoint (address) comparators and two watchpoint

(data) comparators.

1.1.2	Flash

The PSOCTM Automotive 4100S device has a flash module with a flash accelerator, tightly coupled to the CPU to improve average access times from the flash block. The low-power flash block is designed to deliver two wait state (WS) access time at 48 MHz. The flash accelerator delivers 85% of single-cycle SRAM access performance on average.

1.1.3	SRAM

The 8 KB of SRAM are provided with zero wait state access at 48 MHz.

1.1.4	SROM

An 8 KB supervisory ROM that contains boot and configuration routines is provided.

Datasheet	6	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Functional overview

1.2	System resources

1.2.1	Power system

The power system is described in detail in the section “Power” on page 19. It provides assurance that voltage levels are as required for each respective mode and either delays mode entry (for example, on power-on reset (POR)) until voltage levels are as required for proper functionality, or generates resets (for example, on brown-out detection). The PSOCTM Automotive 4100S operates with a single external supply over the range of either 1.8 V ± 5% (externally regulated) or 1.8 V to 5.5 V (internally regulated) and has three different power modes, transitions between which are managed by the power system. The PSOCTM Automotive 4100S provides Active, Sleep, and Deep Sleep low-power modes.

All subsystems are operational in Active mode. The CPU subsystem (CPU, flash, and SRAM) is clock-gated off in Sleep mode, while all peripherals and interrupts are active with instantaneous wake-up on a wake-up event. In Deep Sleep mode, the high-speed clock and associated circuitry is switched off; wake-up from this mode takes 35 µs. The opamps can remain operational in Deep Sleep mode.

1.2.2	Clock system

The PSOCTM Automotive 4100S clock system is responsible for providing clocks to all subsystems that require

clocks and for switching between different clock sources without glitching. In addition, the clock system ensures

that there are no metastable conditions.

The clock system consists of the internal main oscillator (IMO), internal low-speed oscillator (ILO), a 32 kHz watch crystal oscillator (WCO) and provision for an external clock. The clock dividers are provided to generate clocks for peripherals on a fine-grained basis. The fractional dividers are also provided to enable clocking of higher data rates for UARTs.

ILO

LF CL K

WCO

HFCLK

IMO

Divide By

2,4,8

External Clock

HFCLK	Prescaler	 SYSCLK

10X 16-bit

4X 16.5-bit, 1X 24.5-bit

Integer Dividers

Fractional Dividers

Figure 2	Clocking architecture

The HFCLK signal can be divided down to generate synchronous clocks for the analog and digital peripherals. There are eight clock dividers for the PSOCTM Automotive 4100S; two of those are fractional dividers. The 16-bit capability allows flexible generation of fine-grained frequency values and is fully supported in PSOCTM Creator.

1.2.3	IMO clock source

The IMO is the primary source of internal clocking in the PSOCTM 4100S. It is trimmed during testing to achieve the

specified accuracy. The IMO default frequency is 24 MHz and it can be adjusted from 24 to 48 MHz in steps of

4 MHz. The IMO tolerance with Infineon-provided calibration settings is ±2%.

1.2.4	ILO clock source

The ILO is a very low-power, nominally 40-kHz oscillator, which is primarily used to generate clocks for the

watchdog timer (WDT) and peripheral operation in Deep Sleep mode. The ILO-driven counters can be calibrated

to the IMO to improve accuracy. Infineon provides a software component, which does the calibration.

Datasheet	7	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Functional overview

1.2.5	Watch crystal oscillator (WCO)

The Infineon’s 32-bit microcontroller, PSOCTM Automotive 4100S clock subsystem also implements a low-frequency (32 kHz watch crystal) oscillator that can be used for precision timing applications.

1.2.6	Watchdog timer

A watchdog timer is implemented in the clock block running from the ILO; this allows watchdog operation during Deep Sleep and generates a watchdog reset if not serviced before the set timeout occurs. The watchdog reset is recorded in a Reset Cause register, which is firmware readable.

1.2.7	Reset

The PSOCTM Automotive 4100S MCU can be reset from a variety of sources including a software reset. The reset events are asynchronous and guarantee reversion to a known state. The reset cause is recorded in a register, which is sticky through reset and allows software to determine the cause of the reset. An XRES pin is reserved for external reset by asserting it active LOW. The XRES pin has an internal pull-up resistor that is always enabled.

1.3	Analog blocks

1.3.1	12-bit successive approximation register (SAR) ADC

The 12-bit, 1-Msps SAR ADC can operate at a maximum clock rate of 18 MHz and requires a minimum of 18 clocks at that frequency to do a 12-bit conversion.

The sample-and-hold (S/H) aperture is programmable allowing the gain bandwidth requirements of the amplifier

driving the SAR inputs, which determine its settling time, to be relaxed if required. It is possible to provide an

external bypass (through a fixed pin location) for the internal reference amplifier.

The SAR is connected to a fixed set of pins through an eight input sequencer. The sequencer cycles through selected channels autonomously (sequencer scan) with zero switching overhead (that is, aggregate sampling bandwidth is equal to 1 Msps whether it is for a single channel or distributed over several channels). The sequencer switching is effected through a state machine or through firmware driven switching. A feature provided by the sequencer is buffering of each channel to reduce CPU interrupt service requirements. To accommodate signals with varying source impedance and frequency, it is possible to have different sample times programmable for each channel. Also, signal range specification through a pair of range registers (low and high range values) is implemented with a corresponding out-of-range interrupt if the digitized value exceeds the programmed range; this allows fast detection of out-of-range values without the necessity of having to wait for a sequencer scan to be completed and the CPU to read the values and check for out-of-range values in software.

The SAR is not available in Deep Sleep mode as it requires a high-speed clock (up to 18 MHz). The SAR operating range is 1.71 V to 5.5 V.

AHB System Bus and Programmable Logic
Interconnect

SAR Sequencer

|  | Sequencing and Control |
| --- | --- |

SARMUX Port (8 inputs)

P7	P0

SARMUX

vminus vplus

NEG

POS

VDDA/2 VDDA VREF

SAR ADC

Reference Selection

Inputs from other Ports

Data and Status Flags

External Reference and Bypass (optional)

Figure 3	SAR ADC

Datasheet	8	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Functional overview

1.3.2	Two opamps (Continuous-time block; CTB)

The PSOCTM Automotive 4100S microcontroller has two opamps with Comparator modes which allow most common analog functions to be performed on-chip eliminating external components; PGAs, voltage buffers, filters, trans-impedance amplifiers, and other functions can be realized, in some cases with external passives. saving power, cost, and space. The on-chip opamps are designed with enough bandwidth to drive the Sample-and-Hold circuit of the ADC without requiring external buffering.

1.3.3	Low-power comparators (LPC)

The PSOCTM Automotive 4100S MCU has a pair of low-power comparators, which can also operate in Deep Sleep modes. This allows the analog system blocks to be disabled while retaining the ability to monitor external voltage levels during low-power modes. The comparator outputs are normally synchronized to avoid metastability unless operating in an asynchronous power mode where the system wake-up circuit is activated by a comparator switch event. The LPC outputs can be routed to pins.

1.3.4	Current DACs

The PSOCTM Automotive 4100S MCU has two IDACs, which can drive any of the pins on the chip. These IDACs have programmable current ranges.

1.3.5	Analog multiplexed buses

The PSOCTM Automotive 4100S MCU has two concentric independent buses that go around the periphery of the chip. These buses (called amux buses) are connected to firmware-programmable analog switches that allow the chip’s internal resources (IDACs, comparator) to connect to any pin on the I/O Ports.

1.3.6	Programmable digital blocks

The programmable I/O (Smart I/O) block is a fabric of switches and LUTs that allows Boolean functions to be performed in signals being routed to the pins of a GPIO port. The Smart I/O can perform logical operations on input pins to the chip and on signals going out as outputs.

1.4	Fixed function digital

1.4.1	Timer/counter/PWM (TCPWM) block

The TCPWM block consists of a 16-bit counter with user-programmable period length. There is a capture register to record the count value at the time of an event (which may be an I/O event), a period register that is used to either stop or auto-reload the counter when its count is equal to the period register, and compare registers to generate compare value signals that are used as PWM duty cycle outputs. The block also provides true and complementary outputs with programmable offset between them to allow use as dead-band programmable complementary PWM outputs. It also has a Kill input to force outputs to a predetermined state; for example, this is used in motor drive systems when an over-current state is indicated and the PWM driving the FETs needs to be shut off immediately with no time for software intervention. PSOCTM Automotive 4100SMCU has five TCPWM blocks in the.

1.4.2	Serial communication block (SCB)

The 32-bit microcontroller, PSOCTM Automotive 4100S has three serial communication blocks, which can be programmed to have SPI, I2C, UART or LIN Slave functionality.

- I2C mode: The hardware I2C block implements a full multi-master and slave interface (it is capable of multi-master arbitration).

This block is capable of operating at speeds of up to 1 Mbps (Fast Mode Plus) and has flexible buffering options to reduce interrupt overhead and latency for the CPU. It also supports EZI2C that creates a mailbox address range in the memory of the PSOCTM Automotive 4100S and effectively reduces I2C communication to reading from and writing to an array in memory. In addition, the block supports an eight deep FIFO for receive and transmit which, by increasing the time given for the CPU to read data, greatly reduces the need for clock stretching caused by the CPU not having read data on time.

Datasheet	9	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Functional overview

The I2C peripheral is compatible with the I2C Standard mode and Fast mode devices as defined in the NXP I2C-bus specification and user manual (UM10204). The I2C bus I/O is implemented with GPIO in open-drain modes.

The PSOCTM Automotive 4100S MCU is not completely compliant with the I2C spec in the following respect:

GPIO cells are not overvoltage tolerant ; therefore, cannot be hot-swapped or powered up independently of the rest of the I2C system.

- UART mode: This is a full-feature UART operating at up to 1 Mbps. It supports automotive single-wire interface (LIN), infrared interface (IrDA), and SmartCard (ISO7816) protocols, all of which are minor variants of the basic UART protocol. In addition, it supports the 9-bit multiprocessor mode that allows addressing of peripherals connected over common RX and TX lines. Common UART functions such as parity error, break detect, and frame error are supported. An 8-level-deep FIFO allows much greater CPU service latencies to be tolerated.

- SPI mode: The SPI mode supports full Motorola SPI, TI SSP (adds a start pulse used to synchronize SPI Codecs), and National Microwire (half-duplex form of SPI). The SPI block can use the FIFO.

1.5	LIN slave mode

The LIN Slave mode uses the SCB hardware block and implements a full LIN slave interface. This LIN Slave is compliant with LIN v1.3, v2.1/2.2, ISO 17987-6, and SAE J2602-2 specification standards. It is certified by C&S GmbH-based on the standard protocol and data link layer conformance tests. The LIN slave can be operated at baud rates of up to ~20 Kbps with a maximum of 40-meter cable length. The PSOCTM Creator software supports up to two LIN slave interfaces in the PSOCTM 4 device, providing built-in application programming interfaces (APIs) based on the LIN specification standard.

1.6	GPIO

The PSOCTM 4100S has up to 38 GPIOs.

The GPIO block implements the following:

- Eight drive modes:

- Analog input mode (input and output buffers disabled)

- Input only

- Weak pull-up with strong pull-down

- Strong pull-up with weak pull-down

- Open drain with strong pull-down

- Open drain with strong pull-up

- Strong pull-up with strong pull-down

- Weak pull-up with weak pull-down

- Input threshold select (CMOS or LVTTL)

- Individual control of input and output buffer enabling/disabling in addition to the drive strength modes

- Selectable slew rates for dV/dt related noise control to improve EMI

The pins are organized in logical entities called ports, which are 8-bit in width. During power-on and reset, the blocks are forced to the disabled state so as not to crowbar any inputs and/or cause excess turn-on current. A multiplexing network known as a high-speed I/O matrix is used to multiplex between various signals that may connect to an I/O pin.

Data output and pin state registers store, respectively, the values to be driven on the pins and the states of the pins themselves.

Every I/O pin can generate an interrupt if so enabled and each I/O port has an interrupt request (IRQ) and interrupt service routine (ISR) vector associated with it (5 for PSOCTM Automotive 4100S microcontroller).

Datasheet	10	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Functional overview

1.7	Special function peripherals

1.7.1	CAPSENSETM

CAPSENSETM is supported in the PSOCTM Automotive 4100S MCU through a CSD block that can be connected to any pins through an analog multiplex bus via analog switches. CAPSENSETM function can thus be provided on any available pin or group of pins in a system under software control. The PSOCTM Creator component is provided for the CAPSENSETM block to make it easy for the user.

The shield voltage can be driven on another analog multiplex bus to provide water-tolerance capability. Water

tolerance is provided by driving the shield electrode in phase with the sense electrode to keep the shield

capacitance from attenuating the sensed input. Proximity sensing can also be implemented.

The CAPSENSETM block has two IDACs, which can be used for general purposes if CAPSENSETM is not being used (both IDACs are available in that case) or if CAPSENSETM is used without water tolerance (one IDAC is available).

The CAPSENSETM block also provides a 10-bit slope ADC function which can be used in conjunction with the CAPSENSETM function.

The CAPSENSETM block is an advanced, low-noise, programmable block with programmable voltage references and current source ranges for improved sensitivity and flexibility. It can also use an external reference voltage. It has a full-wave CSD mode that alternates sensing to VDDA and ground to null out power-supply related noise.

1.7.2	LCD segment drive

The 32-bit microcontroller, PSOCTM Automotive 4100S has an LCD controller, which can drive up to four commons and up to 32 segments. It uses full digital methods to drive the LCD segments requiring no generation of internal LCD voltages. The two methods used are referred to as Digital Correlation and PWM. Digital Correlation pertains to modulating the frequency and drive levels of the common and segment signals to generate the highest RMS voltage across a segment to light it up or to keep the RMS signal to zero. This method is good for STN displays but may result in reduced contrast with TN (cheaper) displays. PWM pertains to driving the panel with PWM signals to effectively use the capacitance of the panel to provide the integration of the modulated pulse-width to generate the desired LCD voltage. This method results in higher power consumption but can result in better results when driving TN displays. LCD operation is supported during Deep Sleep refreshing a small display buffer (4 bits; 1 32-bit register per port).

Datasheet	11	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Pinouts for 32-bit PSOCTM Automotive 4100S microcontrollers

2	Pinouts for 32-bit PSOCTM Automotive 4100S microcontrollers

Table 1 provides the pin list for PSOCTM Automotive 4100S for the for the 40-pin QFN, and 28-pin SSOP packages. All port pins support GPIO.

Table 1	Pin list

| 40-pin QFN-Auto |  | 28-pin SSOP-Auto |  |
| --- | --- | --- | --- |
| Pin | Name | Pin | Name |
| 22 | P0.0 | 19 | P0.0 |
| 23 | P0.1 | 20 | P0.1 |
| 24 | P0.2 | 21 | P0.2 |
| 25 | P0.3 | 22 | P0.3 |
| 26 | P0.4 | – | – |
| 27 | P0.5 | – | – |
| 28 | P0.6 | 23 | P0.6 |
| 29 | P0.7 | 24 | P0.7 |
| 30 | XRES | 25 | XRES |
| 31 | VCCD | 26 | VCCD |
| 32 | VSSD | 27 | VSS |
| 33 | VDD | 28 | VDD |
| 33 | VDD | – | – |
| 34 | VSSA | – | – |
| 35 | P1.0 | 1 | P1.0 |
| 36 | P1.1 | 2 | P1.1 |
| 37 | P1.2 | 3 | P1.2 |
| 38 | P1.3 | 4 | P1.3 |
| 39 | P1.4 | 5 | P1.4 |
| 40 | P1.7/VREF | 6 | P1.7/VREF |
| 1 | P2.0 | – | – |
| 2 | P2.1 | – | – |
| 3 | P2.2 | – | – |
| 4 | P2.3 | – | – |
| 5 | P2.4 | 7 | P2.4 |
| 6 | P2.5 | 8 | P2.5 |
| 7 | P2.6 | 9 | P2.6 |
| 8 | P2.7 | 10 | P2.7 |
| 9 | VSSD | – | – |
| 10 | P3.0 | 11 | P3.0 |
| 11 | P3.1 | 12 | P3.1 |
| 12 | P3.2 | 13 | P3.2 |
| 13 | P3.3 | 14 | P3.3 |
| 14 | P3.4 | – | – |

Datasheet	12	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Pinouts for 32-bit PSOCTM Automotive 4100S microcontrollers

Table 1	Pin list (continued)

| 40-pin QFN-Auto |  | 28-pin SSOP-Auto |  |
| --- | --- | --- | --- |
| Pin | Name | Pin | Name |
| 15 | P3.5 | – | – |
| 16 | P3.6 | – | – |
| 17 | P3.7 | – | – |
| – | – | – | – |
| 18 | P4.0 | 15 | P4.0 |
| 19 | P4.1 | 16 | P4.1 |
| 20 | P4.2 | 17 | P4.2 |
| 21 | P4.3 | 18 | P4.3 |

The power pins description is as follows:

- VDDD: Power supply for the digital section

- VDDA: Power supply for the analog section

- VSSD, VSSA: Ground pins for the digital and analog sections respectively

- VCCD: Regulated digital supply (1.8 V ± 5%)

- VDD: On some packages, VDDA and VDDD are shorted inside and brought out as a single power supply

- NC: No connection

VDD

VSS

VCCD

P1.0 P1.1 P1.2 P1.3

P1.4

XRES

P0.7

P0.6

P1.7/ VREF

P2.4

P0.3

P2.5

28 - SSOP

P0.2

P0.1

P2.6

28 27 26 25 24 23 22 21 20

19

P2.7

P0.0

P4.3

P3.0

P4.2

P3.1

P4.1

11

12

13

P3.2

P3.3

P4.0

18

17

16

15

1

2

3

4

5

6

7

8

9

10

14

Figure 4	28PIN_SSOP

Datasheet	13	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Pinouts for 32-bit PSOCTM Automotive 4100S microcontrollers

| 7/ VREF 1 P . | P1.1 P1.2 P1.3 P1.4 | P1.0 | VDD VSSA | VSSD | VCC D |
| --- | --- | --- | --- | --- | --- |

1

XRES P0.7 P0.6 P0.5 P0.4 P0.3 P0.2 P0.1

P0.0

40 - QFN

P2.0 P2.1 P2.2 P2.3 P2.4 P2.5 P2.6 P2.7 VSSD

P3.0

30 29 28 27 26 25 24 23 22

21

P4.3

2

3

4

5

6

7

8

9

10

| 3 1 | P3.2 | P3.3 | P3.4 | P3.5 | P3.6 | P3.7 | P4.0 | P4.1 | P4.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

40

39

38

37

36

35

34

33

32

31

1

12

13

14

15

16

17

18

19

20

Figure 5	40PIN_QFN

Datasheet	14	002-15106 Rev. *L

2025-06-06

PSOCTM Automotive 4100S

Based on Arm® Cortex®-M0+ CPU

Pinouts for 32-bit PSOCTM Automotive 4100S microcontrollers

Datasheet	15	002-15106 Rev. *L

2025-06-06

2.1	Alternate pin functions

Each port pin has can be assigned to one of multiple functions. For example, it can be an analog I/O, a digital peripheral function, an LCD pin, or a CAPSENSETM pin.

Table 2 provides the pin assignments.

Table 2	Alternate pin functions

| 40-QFN-PIN | 28-pin SSOP | Name | Analog | Smart I/O | Alternate function 1 | Alternate function 2 | Alternate function 3 | Deep Sleep 1 | Deep Sleep 2 | Deep Sleep 3 | Deelp Sleep 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 22 | 19 | P0.0 | lpcomp.in_p[0] | – | – | scb[2].uart_cts:0 | tcpwm.tr_in[0] | lcd.com[0] | lcd.seg[0] | scb[2].i2c_scl:0 | scb[0].spi_se-lect1:0 |
| 23 | 20 | P0.1 | lpcomp.in_n[0] | – | – | scb[2].uart_rts:0 | tcpwm.tr_in[1] | lcd.com[1] | lcd.seg[1] | scb[2].i2c_sda:0 | scb[0].spi_se-lect2:0 |
| 24 | 21 | P0.2 | lpcomp.in_p[1] | – | – | – | – | lcd.com[2] | lcd.seg[2] | – | scb[0].spi_se-lect3:0 |
| 25 | 22 | P0.3 | lpcomp.in_n[1] | – | – | – | – | lcd.com[3] | lcd.seg[3] | – | scb[2].spi_select0 |
| 26 | – | P0.4 | wco.wco_in | – | – | scb[1].uart_rx:0 | scb[2].uart_rx:0 | lcd.com[4] | lcd.seg[4] | scb[1].i2c_scl:0 | scb[1].spi_mosi:1 |
| 27 | – | P0.5 | wco.wco_out | – | – | scb[1].uart_tx:0 | scb[2].uart_tx:0 | lcd.com[5] | lcd.seg[5] | scb[1].i2c_sda:0 | scb[1].spi_miso:1 |
| 28 | 23 | P0.6 | srss.adft_por_pad _hv | – | srss.ext_clk | scb[1].uart_cts:0 | scb[2].uart_tx:1 | lcd.com[6] | lcd.seg[6] | – | scb[1].spi_clk:1 |
| 29 | 24 | P0.7 | – | – | tcpwm.line[0]:2 | scb[1].uart_rts:0 | – | lcd.com[7] | lcd.seg[7] | – | scb[1].spi_se-lect0:1 |
| 35 | 1 | P1.0 | pass.ctb0_pads[0] | – | tcpwm.line[2]:1 | scb[0].uart_rx:1 | – | lcd.com[8] | lcd.seg[8] | scb[0].i2c_scl:0 | scb[0].spi_mosi:1 |
| 36 | 2 | P1.1 | pass.ctb0_pads[1] | – | tcpwm.line_ compl[2]:1 | scb[0].uart_tx:1 | – | lcd.com[9] | lcd.seg[9] | scb[0].i2c_sda:0 | scb[0].spi_miso:1 |
| 37 | 3 | P1.2 | pass.ctb0_pads[2] pass.ctb0_oa0_ou t_10x | – | tcpwm.line[3]:1 | scb[0].uart_cts:1 | tcpwm.tr_in[2] | lcd.com[10] | lcd.seg[10] | scb[2].i2c_scl:1 | scb[0].spi_clk:1 |
| 38 | 4 | P1.3 | pass.ctb0_pads[3] pass.ctb0_oa1_ou t_10x | – | tcpwm.line_ compl[3]:1 | scb[0].uart_rts:1 | tcpwm.tr_in[3] | lcd.com[11] | lcd.seg[11] | scb[2].i2c_sda:1 | scb[0].spi_se-lect0:1 |
| 39 | 5 | P1.4 | pass.ctb0_pads[4] | – | – | – | – | lcd.com[12] | lcd.seg[12] | – | scb[0].spi_se-lect1:1 |
| 40 | 6 | P1.7/VREF | pass.ctb0_pads[7] pass.sar_ext_vref0 pass.sar_ext_vref1 | – | – | – | – | lcd.com[15] | lcd.seg[15] | – | scb[2].spi_clk |
| 1 | – | P2.0 | pass.sarmux- _pads[0] | prgio[0].io[0] | tcpwm.line[4]:0 | csd.comp | tcpwm.tr_in[4] | lcd.com[16] | lcd.seg[16] | scb[1].i2c_scl:1 | scb[1].spi_mosi:2 |
| 2 | – | P2.1 | pass.sarmux- _pads[1] | prgio[0].io[1] | tcpwm.line_ compl[4]:0 | – | tcpwm.tr_in[5] | lcd.com[17] | lcd.seg[17] | scb[1].i2c_sda:1 | scb[1].spi_miso:2 |
| 3 | – | P2.2 | pass.sarmux-_pads[2] | prgio[0].io[2] | – | – | – | lcd.com[18] | lcd.seg[18] | – | scb[1].spi_clk:2 |

PSOCTM Automotive 4100S

Based on Arm® Cortex®-M0+ CPU

Pinouts for 32-bit PSOCTM Automotive 4100S microcontrollers

Datasheet	16	002-15106 Rev. *L

2025-06-06

Table 2	Alternate pin functions (continued)

| 40-QFN-PIN | 28-pin SSOP | Name | Analog | Smart I/O | Alternate function 1 | Alternate function 2 | Alternate function 3 | Deep Sleep 1 | Deep Sleep 2 | Deep Sleep 3 | Deelp Sleep 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | – | P2.3 | pass.sarmux- _pads[3] | prgio[0].io[3] |  | – | – | tcd.com[19] | tcd.seg[19] | – | scb[1].spi_se-tect0:2 |
| 5 | 7 | P2.4 | pass.sarmux- _pads[4] | prgio[0].io[4] | tcpwm.tine[0]:1 | – | – | tcd.com[20] | tcd.seg[20] | – | scb[1].spi_se-tect1:1 |
| 6 | 8 | P2.5 | pass.sarmux- _pads[5] | prgio[0].io[5] | tcpwm.tine_ compt[0]:1 | – | – | tcd.com[21] | tcd.seg[21] | – | scb[1].spi_se-tect2:1 |
| 7 | 9 | P2.6 | pass.sarmux- _pads[6] | prgio[0].io[6] | tcpwm.tine[1]:1 | – | – | tcd.com[22] | tcd.seg[22] | – | scb[1].spi_se-tect3:1 |
| 8 | 10 | P2.7 | pass.sarmux- _pads[7] | prgio[0].io[7] | tcpwm.tine_ compt[1]:1 | – | – | tcd.com[23] | tcd.seg[23] | tpcomp.comp[0]:1 | scb[2].spi_mosi |
| 10 | 11 | P3.0 | – | prgio[1].io[0] | tcpwm.tine[0]:0 | scb[1].uart_rx:1 | – | tcd.com[24] | tcd.seg[24] | scb[1].i2c_sct:2 | scb[1].spi_mosi:0 |
| 11 | 12 | P3.1 | – | prgio[1].io[1] | tcpwm.tine_ compt[0]:0 | scb[1].uart_tx:1 | – | tcd.com[25] | tcd.seg[25] | scb[1].i2c_sda:2 | scb[1].spi_miso:0 |
| 12 | 13 | P3.2 | – | prgio[1].io[2] | tcpwm.tine[1]:0 | scb[1].uart_cts:1 | – | tcd.com[26] | tcd.seg[26] | cpuss.swd_data | scb[1].spi_ctk:0 |
| 13 | 14 | P3.3 | – | prgio[1].io[3] | tcpwm.tine_ compt[1]:0 | scb[1].uart_rts:1 | – | tcd.com[27] | tcd.seg[27] | cpuss.swd_ctk | scb[1].spi_se-tect0:0 |
| 14 | – | P3.4 | – | prgio[1].io[4] | tcpwm.tine[2]:0 | – | tcpwm.tr_in[6] | tcd.com[28] | tcd.seg[28] | – | scb[1].spi_se-tect1:0 |
| 15 | – | P3.5 | – | prgio[1].io[5] | tcpwm.tine_ compt[2]:0 | – | – | tcd.com[29] | tcd.seg[29] | – | scb[1].spi_se-tect2:0 |
| 16 | – | P3.6 | – | prgio[1].io[6] | tcpwm.tine[3]:0 | – | – | tcd.com[30] | tcd.seg[30] | – | scb[1].spi_se-tect3:0 |
| 17 | – | P3.7 | – | prgio[1].io[7] | tcpwm.tine_ compt[3]:0 | – | – | tcd.com[31] | tcd.seg[31] | tpcomp.comp[1]:1 | scb[2].spi_miso |
| 18 | 15 | P4.0 | csd.vref_ext | – | – | scb[0].uart_rx:0 | – | tcd.com[32] | tcd.seg[32] | scb[0].i2c_sct:1 | scb[0].spi_mosi:0 |
| 19 | 16 | P4.1 | csd.cshietd / csd.cmod | – | – | scb[0].uart_tx:0 | – | tcd.com[33] | tcd.seg[33] | scb[0].i2c_sda:1 | scb[0].spi_miso:0 |
| 20 | 17 | P4.2 | csd.cmod csd.CintA | – | – | scb[0].uart_cts:0 | – | tcd.com[34] | tcd.seg[34] | tpcomp.comp[0]:0 | scb[0].spi_ctk:0 |
| 21 | 18 | P4.3 | csd.csh_tank csd.csh_CintB | – | – | scb[0].uart_rts:0 | – | tcd.com[35] | tcd.seg[35] | tpcomp.comp[1]:0 | scb[0].spi_se-tect0:0 |

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Pinouts for 32-bit PSOCTM Automotive 4100S microcontrollers

2.2	Function description

Table 4 provides the description for each function name used in Table 2.
Table 3	PSOCTM Automotive 4100S function description

| Signal name | Description | I/O |
| --- | --- | --- |
| lpcomp.in_p[x] | Low Power Comparator Input - Positive of Block [X] | I |
| lpcomp.in_n[x] | Low Power Comparator Input - Negative of Block [X] | I |
| WCO.wco_in | Watch Crystal Oscillator Input | I |
| WCO.wco_out | Watch Crystal Oscillator Output | O |
| pass.ctb0_pads[X] | Continuous Time Block (OPAMP) Input | I |
| pass.ctb0_oa0_out_10x | Continuous Time Block (OPAMP) Output with 10x gain | O |
| pass.sar_ext_verf0 | SAR ADC external Vref voltage | I |
| pass.sarmux_pads[Y] | SAR ADC MUX pads [Y] | I |
| csd.vref_ext | CapSense External Vref Voltage | I |
| csd.cshieldpads | CapSense Shield connections (all other IO pins are also supported for shield) | O |
| csd.cmodpad | CapSense CMOD capacitor for CSD | I/O |
| csd.csh_tank | CapSense Tank Capacitor for Shield (optional) | I/O |
| csd.cintA | CapSense CintA Capacitor for CSX operation | I/O |
| csd.cintA | CapSense CintB capacitor for CSX operation | I/O |
| prgio[x].io[y] | Programmable/Smart IO Block [X] - IO Pin [Y] | I/O |
| srss.ext_clk | External clock input | I |
| tcpwm.line[X] | Timer Counter PWM (TCPWM) Block [X] - Output | O |
| tcpwm.line_compl[X] | Timer Counter PWM (TCPWM) Block [X] - Output Complement | O |
| SCB[x].uart_rx | Serial Communication Block [X] - UART Receive | I |
| SCB[x].uart_tx | Serial Communication Block [X] - UART Transmit | O |
| SCB[x].uart_rts | Serial Communication Block [X] - UART Ready To Send | O |
| SCB[x].uart_cts | Serial Communication Block [X] - UART Clear To Send | O |
| csd.comp | CapSense Comparator Output | O |
| tcpwm.tr_in[y] | Timer Counter PWM (TCPWM) Block trigger input [Y] - for Reload/Start/Stop/Count/Capture | I |
| lcd.com[X] | LDC Drive Common pad [X] | O |
| lcd.seg[X] | LDC Drive Segment pad [X] | O |
| SCB[x].i2c_scl | Serial Communication Block [X] - I2C SCL | I/O |
| SCB[x].i2c_sda | Serial Communication Block [X] - I2C SDA | I/O |
| lpcomp.comp[X] | Low Power Comparator Output for Block [X] | O |

Datasheet	17	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Pinouts for 32-bit PSOCTM Automotive 4100S microcontrollers

Table 3	PSOCTM Automotive 4100S function description

| Signal name | Description | I/O |
| --- | --- | --- |
| cpuss.swd_data | SWD_Data (Programming/Debug) | I/O |
| cpuss.swd_clk | SWD_CLK (Programming/Debug) | I/O |
| scb[x].spi_select | Serial Communication Block - SPI select | O |
| SCB[x].spi_mosi | Serial Communication Block [X] - SPI MOSI | I/O |
| SCB[x].spi_miso | Serial Communication Block [X] - SPI MISO | I/O |
| SCB[x].spi_clk | Serial Communication Block [X] - SPI Clock | I/O |

Datasheet	18	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Power

3	Power

Figure 6 illustrates the set of power supply pins as implemented for the PSOCTM Automotive 4100S MCU. The

system has one regulator in Active mode for the digital circuitry. There is no analog regulator; the analog circuits

run directly from the VDDA input.

VDDA

VSSA

VDDD

VDDD

VSSD

VDDA

VCCD

1.8 V Regulator

Analog Domain

Digital Domain

Figure 6	Power supply connections

There are two distinct modes of operation. In Mode 1, the supply voltage range is 1.8 V to 5.5 V (unregulated

externally; internal regulator operational). In Mode 2, the supply range is 1.8 V ± 5% (externally regulated;

1.71 V to 1.89 V, internal regulator bypassed).

3.1	Mode 1: 1.8 V to 5.5 V external supply

In this mode, the 32-bit microcontroller, PSOCTM Automotive 4100S is powered by an external power supply that can be anywhere in the range of

1.8 V to 5.5 V. This range is also designed for battery-powered operation. For example, the chip can be powered from a battery system that starts at 3.5 V and works down to 1.8 V. In this mode, the internal regulator of the PSOCTM 4100S supplies the internal logic and its output is connected to the VCCD pin. The VCCD pin must be bypassed to ground via an external capacitor (0.1 µF; X5R ceramic or better) and must not be connected to anything else.

3.2	Mode 2: 1.8 V ± 5% external supply

In this mode, the 32-bit microcontroller, PSOCTM Automotive 4100S is powered by an external power supply that must be within the range of 1.71 V to 1.89 V; note that this range needs to include the power supply ripple too. In this mode, the VDD and VCCD pins are shorted together and bypassed.

Bypass capacitors must be used from VDDD to ground. The typical practice for systems in this frequency range is to use a capacitor in the 1-µF range, in parallel with a smaller capacitor (for example, 0.1 µF). Note that these are simply rules of thumb and that, for critical applications, the PCB layout, lead inductance, and the bypass capacitor parasitic should be simulated to design and obtain optimal bypassing.

On some packages, VDDD and VDDA pins are shorted inside the package and brought out as a generic VDD pin. In that case, only 0.1 µF and 1 µF decoupling capacitors are required on the VDD pin. Figure 7 illustrates an example of a bypass scheme.

Datasheet	19	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Power

1mF

1.8 V to 5.5 V

1.8 V to 5.5 V

0.1mF

1mF

VDD

VDDA

0.1mF

VCCD

PSOCTM Automotive 4100S

0.1mF

VSS

Figure 7	External supply range from 1.8 V to 5.5 V with internal regulator active

Datasheet	20	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Development support

4	Development support

The PSOCTM Automotive 4100S family has a rich set of documentation, development tools, and online resources to assist you during your development process. Visit www.infineon.com/psoc4 to find out more.

4.1	Documentation

A suite of documentation supports the PSOCTM Automotive 4100S family to ensure that you can find answers to your questions quickly. This section contains a list of some of the key documents.

Software user guide: A step-by-step guide for using PSOCTM Creator. The software user guide shows you how the PSOCTM Creator build process works in detail, how to use source control with PSOCTM Creator, and much more.

Component datasheets: The flexibility of PSOCTM allows the creation of new peripherals (components) long after the device has gone into production. Component data sheets provide all of the information needed to select and use a particular component, including a functional description, API documentation, example code, and AC/DC specifications.

Application notes: PSOCTM application notes discuss a particular application of PSOCTM in depth; examples

include brushless DC motor control and on-chip filtering. The application notes often include example projects

in addition to the application note document.

Reference manual: The Reference Manual contains all the technical detail you need to use a PSOCTM device,

including a complete description of all PSOCTM registers. The reference manual is available in the Documentation

section at www.infineon.com/psoc4.

4.2	Online

In addition to print documentation, the PSOCTM forums connect you with fellow PSOCTM users and experts in PSOCTM from around the world, 24 hours a day, 7 days a week.

4.3	Tools

With industry standard cores, programming, and debugging interfaces, the PSOCTM Automotive 4100S family is part of a development tool ecosystem. Visit us at www.infineon.com/psoccreator for the latest information on the revolutionary, easy to use PSOCTM Creator IDE, supported third party compilers, programmers, debuggers, and development kits.

Datasheet	21	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Electrical specifications

5	Electrical specifications

5.1	Absolute maximum ratings

Table 4	Absolute maximum ratings[2]

| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SID1 | VDDD_ABS | Digital supply relative to VSS | –0.5 | – | 6 | V | – |
| SID2 | VCCD_ABS | Direct digital core voltage input relative to VSS | –0.5 | – | 1.95 |  | – |
| SID3 | VGPIO_ABS | GPIO voltage | –0.5 | – | VDD + 0.5 |  | – |
| SID4 | IGPIO_ABS | Maximum current per GPIO | –25 | – | 25 | mA | – |
| SID5 | IGPIO_injection | GPIO injection current, Max for VIH > VDDD, and Min for VIL < VSS | –0.5 | – | 0.5 |  | Current injected per pin |
| BID44 | ESD_HBM | Electrostatic discharge human body model | 2200 | – | – | V | – |
| BID45 | ESD_CDM | Electrostatic discharge charged device model | 500 | – | – |  | – |
| BID46 | LU | Pin current for latch-up | –140 | – | 140 | mA | – |

Note

2. Usage above the absolute maximum conditions listed in Table 4 may cause permanent damage to the device. Exposure to Absolute Maximum conditions for extended periods of time may affect device reliability. The maximum storage temperature is 150°C in compliance with JEDEC Standard JESD22-A103, High Temperature Storage Life. When used below Absolute Maximum conditions but above normal operating conditions, the device may not operate to specification.

Datasheet	22	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Electrical specifications

5.2	Device-level specifications

All specifications are valid for –40°C ≤ TA ≤ 85°C for Grade-A devices, –40°C ≤ TA ≤ 105°C for Grade-S devices, and –40°C ≤ TA ≤ 125°C for Grade-E devices. Specifications are valid for 1.71 V to 5.5 V, except where noted.[3]

5.2.1	DC specifications

Table 5	DC specifications

Typical values measured at VDD = 3.3 V and 25°C.

| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SID53 | VDD | Power supply input voltage | 1.8 | – | 5.5 | V | Internally regulated supply |
| SID255 | VDD | Power supply input voltage (VCCD = VDDD = VDDA) | 1.71 | – | 1.89 |  | Internally unregulated supply |
| SID54 | VCCD | Output voltage (for core logic) | – | 1.8 | – |  | – |
| SID55 | CEFC | External regulator voltage bypass | – | 0.1 | – | µF | X5R ceramic or better |
| SID56 | CEXC | Power supply bypass capacitor | – | 1 | – |  | X5R ceramic or better |

Active Mode, VDD = 1.8 V to 5.5 V. Typical values measured at VDD = 3.3 V and 25°C

| SID10 | IDD5 | Execute from flash; CPU at 6 MHz | – | 1.8 | 2.7 | mA | Max is at 125°C and 5.5 V. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SID16 | IDD8 | Execute from flash; CPU at 24 MHz | – | 3.0 | 4.75 |  | Max is at 125°C and 5.5 V. |
| SID19 | IDD11 | Execute from flash; CPU at 48 MHz | – | 5.4 | 6.85 |  | Max is at 125°C and 5.5 V. |
| Sleep Mode, VDDD = 1.8 V to 5.5 V (Regulator on) |  |  |  |  |  |  |  |
| SID22 | IDD17 | I2C wakeup WDT, and Comparators on | – | 1.7 | 2.2 | mA | 6 MHz. Max is at 125°C and 5.5 V. |
| SID25 | IDD20 | I2C wakeup, WDT, and Comparators on. | – | 2.2 | 2.5 |  | 12 MHz. Max is at 125°C and 5.5 V. |

Sleep Mode, VDDD = 1.71 V to 1.89 V (Regulator bypassed)

| SID28 | IDD23 | I2C wakeup, WDT, and Comparators on | – | 0.7 | 0.9 | mA | 6 MHz. Max is at 125°C and 5.5 V. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SID28A | IDD23A | I2C wakeup, WDT, and Comparators on | – | 1.0 | 1.2 | mA | 12 MHz. Max is at 125°C and 5.5 V. |

Deep Sleep Mode, VDD = 1.8 V to 3.6 V (Regulator on)

| SID31 | IDD26 | I2C wakeup and WDT on | – | 2.5 | 150 | µA | Max is at 3.6 V and 125°C. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Deep Sleep Mode, VDD = 3.6 V to 5.5 V (Regulator on) |  |  |  |  |  |  |  |
| SID34 | IDD29 | I2C wakeup and WDT on | – | 2.5 | 150 | µA | Max is at 5.5 V and 125°C. |
| Deep Sleep Mode, VDD = VCCD = 1.71 V to 1.89 V (Regulator bypassed) |  |  |  |  |  |  |  |
| SID37 | IDD32 | I2C wakeup and WDT on | – | 2.5 | 160 | µA | Max is at 1.89 V and 125°C. |
| XRES Current |  |  |  |  |  |  |  |
| SID307 | IDD_XR | Supply current while XRES asserted | – | 2 | 5 | mA | – |

Notes

3. This device is not AEC-Q100 Grade 0 qualified, so Infineon does not guarantee performance at +150°C. The

specifications for 125°C < TA £ 150°C are best estimates of the performance.

Datasheet	23	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Electrical specifications

5.2.2	AC specifications

Table 6	AC specifications

| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SID48 | FCPU | CPU frequency | DC | – | 48 | MHz | 1.71 V £ VDD £ 5.5 V |
| SID49[4] | TSLEEP | Wakeup from Sleep mode | – | 0 | – | µs | – |
| SID50[4] | TDEEPSLEEP | Wakeup from Deep Sleep mode | – | 35 | – |  | – |

5.2.3	GPIO

Table 7	GPIO DC specifications

| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SID57 | VIH[5] | Input voltage high threshold | 0.7 × VDDD | – | – | V | CMOS Input |
| SID58 | VIL | Input voltage low threshold | – | – | 0.3 × VDDD |  | CMOS Input |
| SID241 | VIH[5] | LVTTL input, VDDD < 2.7 V | 0.7 × VDDD | – | – |  | – |
| SID242 | VIL | LVTTL input, VDDD < 2.7 V | – | – | 0.3 × VDDD |  | – |
| SID243 | VIH[5] | LVTTL input, VDDD ³ 2.7 V | 2.0 | – | – |  | – |
| SID244 | VIL | LVTTL input, VDDD ³ 2.7 V | – | – | 0.8 |  | – |
| SID59 | VOH | Output voltage high level | VDDD – 0.6 | – | – |  | IOH = 4 mA at 3 V VDDD |
| SID60 | VOH | Output voltage high level | VDDD – 0.5 | – | – |  | IOH = 1 mA at 1.8 V VDDD |
| SID61 | VOL | Output voltage low level | – | – | 0.6 |  | IOL = 4 mA at 1.8 V VDDD |
| SID62 | VOL | Output voltage low level | – | – | 0.6 |  | IOL = 10 mA at 3 V VDDD |
| SID62A | VOL | Output voltage low level | – | – | 0.4 |  | IOL = 3 mA at 3 V VDDD |
| SID63 | RPULLUP | Pull-up resistor | 3.5 | 5.6 | 8.5 | kΩ | –40°C £ TA £ 125°C |
| SID63A | RPULLUP | Pull-up resistor | 3 | – | – |  | 125°C < TA £ 150°C |
| SID64 | RPULLDOWN | Pull-down resistor | 3.5 | 5.6 | 8.5 |  | –40°C £ TA £ 125°C |
| SID64A | RPULLDOWN | Pull-down resistor | 3 | – | – |  | 125°C < TA £ 150°C |
| SID65 | IIL | Input leakage current (absolute value) | – | – | 2 | nA | 25°C, VDDD = 3.0 V |
| SID66 | CIN | Input capacitance | – | – | 7 | pF | – |

Notes

- Guaranteed by characterization.

- VIH must not exceed VDDD + 0.2 V.

Datasheet	24	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Electrical specifications

Table 7	GPIO DC specifications (continued)

| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SID67[6] | VHYSTTL | Input hysteresis LVTTL | 25 | 40 | – | mV | VDDD >- 2.7 V, –40°C ≤ TA ≤ 125°C |
| SID67A[6] | VHYSTTL | Input hysteresis LVTTL | 7.5 | – | – |  | 125°C < TA ≤ 150°C |
| SID68[6] | VHYSCMOS | Input hysteresis CMOS | 0.05 × VDDD | – | – |  | VDD < 4.5 V |
| SID68A[6] | VHYSCMOS5V5 | Input hysteresis CMOS | 200 | – | – |  | VDD > 4.5 V |
| SID69[6] | IDIODE | Current through protection diode to VDD/VSS | – | – | 100 | µA | – |
| SID69A[6] | ITOT_GPIO | Maximum total source or sink chip current | – | – | 200 | mA | – |

5.2.4	XRES

Table 8	GPIO AC specifications (Guaranteed by characterization)

| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SID70 | TRISEF | Rise time in fast strong mode | 2 | – | 12 | ns | 3.3 V VDDD, Cload = 25 pF, –40°C ≤ TA ≤ 125°C |
| SID70A | TRISEF | Rise time in fast strong mode | – | – | 25 |  | 3.3 V VDDD, Cload = 25 pF, 125°C < TA ≤ 150°C |
| SID71 | TFALLF | Fall time in fast strong mode | 2 | – | 12 |  | 3.3 V VDDD, Cload = 25 pF, –40°C ≤ TA ≤ 125°C |
| SID71A | TFALLF | Fall time in fast strong mode | – | – | 25 |  | 3.3 V VDDD, Cload = 25 pF, 125°C < TA ≤ 150°C |
| SID72 | TRISES | Rise time in slow strong mode | 10 | – | 60 | – | 3.3 V VDDD, Cload = 25 pF, –4°C ≤ TA ≤ 125°C |
| SID72A | TRISES | Rise time in slow strong mode | – | – | 130 | ns | 3.3 V VDDD, Cload = 25 pF, 125°C < TA ≤ 150°C |
| SID73 | TFALLS | Fall time in slow strong mode | 10 | – | 60 | – | 3.3 V VDDD, Cload = 25 pF, –40°C ≤ TA ≤ 125°C |
| SID73A | TFALLS | Fall time in slow strong mode | – | – | 130 | ns | 3.3 V VDDD, Cload = 25 pF, 125°C < TA ≤ 150°C |

Notes

6. Guaranteed by characterization.

Datasheet	25	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Electrical specifications

Table 8	GPIO AC specifications (continued) (Guaranteed by characterization)

| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SID74 | FGPIOUT1 | GPIO FOUT; 3.3 V £ VDDD £ 5.5 V Fast strong mode | – | – | 33 | MHz | 90/10%, 25-pF load, 60/40 duty cycle |
| SID75 | FGPIOUT2 | GPIO FOUT; 1.71 V£ VDDD £ 3.3 V Fast strong mode | – | – | 16.7 |  | 90/10%, 25-pF load, 60/40 duty cycle |
| SID76 | FGPIOUT3 | GPIO FOUT; 3.3 V £ VDDD £ 5.5 V Slow strong mode | – | – | 7 |  | 90/10%, 25-pF load, 60/40 duty cycle |
| SID245 | FGPIOUT4 | GPIO FOUT; 1.71 V £ VDDD £ 3.3 V Slow strong mode. | – | – | 3.5 |  | 90/10%, 25-pF load, 60/40 duty cycle |
| SID246 | FGPIOIN | GPIO input operating frequency; 1.71 V £ VDDD £ 5.5 V | – | – | 48 |  | 90/10% VIO |

Table 9	XRES DC specifications

| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SID77 | VIH | Input voltage high threshold | 0.7 × VDDD | – | – | V | CMOS Input |
| SID78 | VIL | Input voltage low threshold | – | – | 0.3 × VDDD |  |  |
| SID79 | RPULLUP | Pull-up resistor | – | 60 | – | kΩ | – |
| SID80 | CIN | Input capacitance | – | – | 7 | pF | – |
| SID81[7' | VHYSXRES | Input voltage hysteresis | – | 100 | – | mV | Typical hysteresis is 200 mV for VDD > 4.5 V |
| SID82 | IDIODE | Current through protection diode to VDD/VSS | – | – | 100 | µA | – |

Table 10	XRES AC specifications

| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SID83[7' | TRESETWIDTH | Reset pulse width | 1 | – | – | µs | – |
| BID194[7' | TRESETWAKE | Wake-up time from reset release | – | – | 2.7 | ms | – |

Note

7. Guaranteed by characterization.

Datasheet	26	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Electrical specifications

5.3	Analog peripherals

5.3.1	CTBm opamp

Table 11	CTBm opamp specifications

| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| – | IDD | Opamp block current, External load | – |  |  |  |  |
| SID269 | IDD_HI | power = hi | – | 1100 | 1850 | µA | – |
| SID270 | IDD_MED | power = med | – | 550 | 950 |  | –40°C £ TA £ 125°C |
| SID270A | IDD_MED | power = med | – | – | 1075 |  | 125°C < TA £150°C |
| SID271 | IDD_LOW | power = lo | – | 150 | 350 |  | –40°C £ TA £ 125°C |
| SID271A | IDD_LOW | power = lo | – | – | 500 |  | 125°C < TA £15 °C |
| – | GBW | Load = 20 pF, 0.1 mA, VDDA = 2.7 V | – |  |  |  |  |
| SID272 | GBW_HI | power = hi | 6 | – | – | MHz | Input and output are 0.2 V to VDDA – 0.2 V, –40°C £ TA £ 125°C |
| SID272A | GBW_HI | power = hi | 4.5 | – | – |  | 125°C < TA £ 150°C |
| SID273 | GBW_MED | power = med | 3 | – | – |  | Input and output are 0.2 V to VDDA – 0.2 V, –40°C £ TA £ 125°C |
| SID273A | GBW_MED | power = med | – | 3 | – |  | 125°C < TA £ 150°C |
| SID274 | GBW_LO | power = lo | – | 1 | – |  | Input and output are 0.2 V to VDDA – 0.2 V |
| – | IOUT_MAX | VDDA = 2.7 V, 500 mV from rail | – |  |  |  |  |
| SID275 | IOUT_MAX_HI | power = hi | 10 | – | – | mA | Output is 0.5 V, VDDA – 0.5 V |
| SID276 | IOUT_MAX_MID | power = mid | 10 | – | – |  | Output is 0.5 V, VDDA – 0.5 V |
| SID277 | IOUT_MAX_LO | power = lo | – | 5 | – |  | Output is 0.5 V, VDDA – 0.5 V |
| – | IOUT | VDDA = 1.71 V, 500 mV from rail | – |  |  |  |  |
| SID278 | IOUT_MAX_HI | power = hi | 4 | – | – | mA | Output is 0.5 V, VDDA – 0.5 V |
| SID279 | IOUT_MAX_MID | power = mid | 4 | – | – |  | Output is 0.5 V, VDDA – 0.5 V |
| SID280 | IOUT_MAX_LO | power = lo | – | 2 | – |  | Output is 0.5 V, VDDA – 0.5 V |

Datasheet	27	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Electrical specifications

Table 11	CTBm opamp specifications (continued)

| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| – | IDD_Int | Opamp block current Internal Load | – |  |  |  |  |
| SID269_I | IDD_HI_Int | power = hi | – | 1500 | 1700 | µA | – |
| SID270_I | IDD_MED_Int | power = med | – | 700 | 900 |  | – |
| SID271_I | IDD_LOW_Int | power = lo | – | – | – |  | – |
| – | GBW | VDDA = 2.7 V | – |  |  |  |  |
| SID272_I | GBW_HI_Int | power = hi | 8 | – | – | MHz | Output is 0.25 V to VDDA – 0.25 V |
| – | – | General opamp specs for both internal and external modes | – |  |  |  |  |
| SID281 | VIN | Charge-pump on, VDDA = 2.7 V | –0.05 | – | VDDA – 0.2 | V | – |
| SID282 | VCM | Charge-pump on, VDDA = 2.7 V | –0.05 | – | VDDA – 0.2 |  | – |
| – | VOUT | VDDA = 2.7 V | – |  |  |  |  |
| SID283 | VOUT_1 | power = hi, Iload = 10 mA | 0.5 | – | VDDA – 0.5 | V | – |
| SID284 | VOUT_2 | power = hi, Iload = 1 mA | 0.2 | – | VDDA – 0.2 |  | – |
| SID285 | VOUT_3 | power = med, Iload = 1 mA | 0.2 | – | VDDA – 0.2 |  | – |
| SID286 | VOUT_4 | power = lo, Iload = 0.1 mA | 0.2 | – | VDDA – 0.2 |  | – |

Datasheet	28	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Electrical specifications

Table 11	CTBm opamp specifications (continued)

| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SID288 | VOS_TR | Offset voltage, trimmed | –1.0 | ±0.5 | 1.0 | mV | High mode, input 0 V to VDDA – 0.2 V, –40°C £ TA £ 125°C |
| SID288C | VOS_TR | Offset voltage, trimmed | –1.3 | – | 1.3 |  | High mode, input 0 V to VDDA – 0.2 V, 125°C < TA £ 150°C |
| SID288A | VOS_TR | Offset voltage, trimmed | – | ±1 | – |  | Medium mode, input 0 V to VDDA – 0.2 V |
| SID288B | VOS_TR | Offset voltage, trimmed | – | ±2 | – |  | Low mode, input 0 V to VDDA – 0.2 V |
| SID290 | VOS_DR_TR | Offset voltage drift, trimmed | –10 | ±3 | 10 | µV/°C | High mode, –40°C £ TA £ 125°C |
| SID290C | VOS_DR_TR | Offset voltage drift, trimmed | –15 | – | 15 |  | High mode, 125°C < TA £ 150°C |
| SID290A | VOS_DR_TR | Offset voltage drift, trimmed | – | ±10 | – |  | Medium mode, –40°C £ TA £ 125°C |
| SID290D | VOS_DR_TR | Offset voltage drift, trimmed | – | ±15 | – |  | Medium mode, 125°C < TA £ 150°C |
| SID290B | VOS_DR_TR | Offset voltage drift, trimmed | – | ±10 | – |  | Low mode, –40°C £ TA £ 125°C |
| SID290E | VOS_DR_TR | Offset voltage drift, trimmed | – | ±15 | – |  | Low mode, 125°C < TA £ 150°C |
| SID291 | CMRR | DC | 70 | 80 | – | dB | Input is 0 V to VDDA – 0.2 V, Output is 0.2 V to VDDA – 0.2 V |
| SID292 | PSRR | At 1 kHz, 10-mV ripple | 70 | 85 | – |  | VDDD = 3.6 V, high-power mode, input is 0.2 V to VDDA – 0.2 V |

Datasheet	29	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Electrical specifications

Table 11	CTBm opamp specifications (continued)

| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| – | – | Noise |  |  |  |  |  |
| SID294 | VN2 | Input-referred, 1 kHz, power = Hi | – | 72 | – | nV/rtHz | Input and output are at 0.2 V to VDDA – 0.2 V |
| SID295 | VN3 | Input-referred, 10 kHz, power = Hi | – | 28 | – |  | Input and output are at 0.2 V to VDDA – 0.2 V |
| SID296 | VN4 | Input-referred, 100 kHz, power = Hi | – | 15 | – |  | Input and output are at 0.2 V to VDDA – 0.2 V |
| SID297 | CLOAD | Stable up to max. load. Performance specs at 50 pF. | – | – | 125 | pF | – |
| SID298 | SLEW_RATE | Cload = 50 pF, Power = High, VDDA = 2.7 V | 6 | – | – | V/µs | – |
| SID299 | T_OP_WAKE | From disable to enable, no external RC dominating | – | – | 25 | µs | – |
| SID299A | OL_GAIN | Open Loop Gain | – | 90 | – | dB | – |
| – | COMP_MODE | Comparator mode; 50 mV drive, Trise = Tfall (approx.) | – |  |  |  |  |
| SID300 | TPD1 | Response time; power = hi | – | 150 | – | ns | Input is 0.2 V to VDDA – 0.2 V |
| SID301 | TPD2 | Response time; power = med | – | 500 | – |  | Input is 0.2 V to VDDA – 0.2 V |
| SID302 | TPD3 | Response time; power = lo | – | 2500 | – |  | Input is 0.2 V to VDDA – 0.2 V |
| SID303 | VHYST_OP | Hysteresis | – | 10 | – | mV | – |
| SID304 | WUP_CTB | Wake-up time from Enabled to Usable | – | – | 25 | µs | – |
| – | Deep Sleep Mode | Mode 2 is lowest current range. Mode 1 has higher GBW. | – |  |  |  |  |

Datasheet	30	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Electrical specifications

Table 11	CTBm opamp specifications (continued)

| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SID_DS_1 | IDD_HI_M1 | Mode 1, High current | – | 1400 | – | µA | 25°C |
| SID_DS_2 | IDD_MED_M1 | Mode 1, Medium current | – | 700 | – |  | 25°C |
| SID_DS_3 | IDD_LOW_M1 | Mode 1, Low current | – | 200 | – |  | 25°C |
| SID_DS_4 | IDD_HI_M2 | Mode 2, High current | – | 120 | – |  | 25°C |
| SID_DS_5 | IDD_MED_M2 | Mode 2, Medium current | – | 60 | – |  | 25°C |
| SID_DS_6 | IDD_LOW_M2 | Mode 2, Low current | – | 15 | – |  | 25°C |
| SID_DS_7 | GBW_HI_M1 | Mode 1, High current | – | 4 | – | MHz | 20-pF load, no DC load 0.2 V to VDDA – 0.2 V |
| SID_DS_8 | GBW_MED_M1 | Mode 1, Medium current | – | 2 | – |  | 20-pF load, no DC load 0.2 V to VDDA – 0.2 V |
| SID_DS_9 | GBW_LOW_M1 | Mode 1, Low current | – | 0.5 | – |  | 20-pF load, no DC load 0.2 V to VDDA – 0.2 V |
| SID_DS_10 | GBW_HI_M2 | Mode 2, High current | – | 0.5 | – |  | 20-pF load, no DC load 0.2 V to VDDA – 0.2 V |
| SID_DS_11 | GBW_MED_M2 | Mode 2, Medium current | – | 0.2 | – |  | 20-pF load, no DC load 0.2 V to VDDA – 0.2 V |
| SID_DS_12 | GBW_Low_M2 | Mode 2, Low current | – | 0.1 | – |  | 20-pF load, no DC load 0.2 V to VDDA – 0.2 V |

Datasheet	31	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Electrical specifications

Table 11	CTBm opamp specifications (continued)

| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SID_DS_13 | VOS_HI_M1 | Mode 1, High current | – | 5 | – | mV | With trim 25°C, 0.2 V to VDDA – 0.2 V |
| SID_DS_14 | VOS_MED_M1 | Mode 1, Medium current | – | 5 | – |  | With trim 25°C, 0.2 V to VDDA – 0.2 V |
| SID_DS_15 | VOS_LOW_M2 | Mode 1, Low current | – | 5 | – |  | With trim 25°C, 0.2 V to VDDA – 0.2 V |
| SID_DS_16 | VOS_HI_M2 | Mode 2, High current | – | 5 | – |  | With trim 25°C, 0.2 V to VDDA – 0.2 V |
| SID_DS_17 | VOS_MED_M2 | Mode 2, Medium current | – | 5 | – |  | With trim 25°C, 0.2 V to VDDA – 0.2 V |
| SID_DS_18 | VOS_LOW_M2 | Mode 2, Low current | – | 5 | – |  | With trim 25°C, 0.2 V to VDDA – 0.2 V |
| SID_DS_19 | IOUT_HI_M1 | Mode 1, High current | – | 10 | – | mA | Output is 0.5 V to VDDA – 0.5 V |
| SID_DS_20 | IOUT_MED_M1 | Mode 1, Medium current | – | 10 | – |  | Output is 0.5 V to VDDA – 0.5 V |
| SID_DS_21 | IOUT_LOW_M1 | Mode 1, Low current | – | 4 | – |  | Output is 0.5 V to VDDA – 0.5 V |
| SID_DS_22 | IOUT_HI_M2 | Mode 2, High current | – | 1 | – |  | – |
| SID_DS_23 | IOU_MED_M2 | Mode 2, Medium current | – | 1 | – |  | – |
| SID_DS_24 | IOU_LOW_M2 | Mode 2, Low current | – | 0.5 | – |  | – |

Datasheet	32	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Electrical specifications

5.3.2	Comparator

Table 12	Comparator DC specifications

| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SID84 | VOFFSET1 | Input offset voltage, Factory trim | – | – | ±10 | mV | Normal mode, –40°C £ TA £ 125°C |
| SID84A | VOFFSET1 | Input offset voltage, Factory trim | – | ±15 | – |  | 125°C < TA £ 150°C, normal mode |
| SID85 | VOFFSET2 | Input offset voltage, Custom trim | – | – | ±4 |  | Low power mode, –40°C £ TA £ 125°C |
| SID85A | VOFFSET2 | Input offset voltage, Custom trim | – | ±4 | – |  | 125°C < TA £ 150°C, low power mode |
| SID86 | VHYST | Hysteresis when enabled | – | 10 | 35 |  | –40°C £ TA £ 125°C |
| SID86A | VHYST | Hysteresis when enabled | – | – | 40 | – | 125°C < TA £ 150°C |
| SID87 | VICM1 | Input common mode voltage in normal mode | 0 | – | VDDD – 0.1 | V | Modes 1 and 2 |
| SID247 | VICM2 | Input common mode voltage in low power mode | 0 | – | VDDD |  | – |
| SID247A | VICM3 | Input common mode voltage in ultra low power mode | 0 | – | VDDD – 1.15 |  | VDDD ≥ 2.2 V at –40°C |
| SID88 | CMRR | Common mode rejection ratio | 50 | – | – | dB | VDDD ≥ 2.7 V, –40°C £ TA £ 125°C |
| SID88B | CMRR | Common mode rejection ratio | – | 50 | – |  | VDDD ≥ 2.7 V; 125°C < TA £ 150°C |
| SID88A | CMRR | Common mode rejection ratio | 42 | – | – |  | VDDD ≤ 2.7 V, –40°C £ TA £ 125°C |
| SID88C | CMRR | Common mode rejection ratio | – | 42 | – |  | VDDD ≤ 2.7 V; 125°C < TA £ 150°C |
| SID89 | ICMP1 | Block current, normal mode | – | – | 400 | µA | – |
| SID248 | ICMP2 | Block current, low power mode | – | – | 100 |  | – |
| SID259 | ICMP3 | Block current in ultra low-power mode | – | – | 6 |  | VDDD ≥ 2.2 V at –40°C |
| SID90 | ZCMP | DC Input impedance of comparator | 35 | – | – | MΩ | – |

Table 13	Comparator AC specifications

| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SID91 | TRESP1 | Response time, normal mode, 50 mV overdrive | – | 38 | 110 | ns | – |
| SID258 | TRESP2 | Response time, low power mode, 50 mV overdrive | – | 70 | 200 |  | – |
| SID92 | TRESP3 | Response time, ultra-low power mode, 200 mV overdrive | – | 2.3 | 15 | µs | VDDD ≥ 2.2 V at –40°C |

Datasheet	33	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Electrical specifications

5.3.3	Temperature sensor

Table 14	Temperature sensor specifications

| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SID93 | TSENSACC | Temperature sensor accuracy | –5 | ±1 | 5 | °C | –40°C to +85°C |
| SID93A | TSENSACC | Temperature sensor accuracy | –15 | – | +15 | °C | –85°C to +150°C |

5.3.4	SAR ADC

Table 15	SAR ADC DC specifications

| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SID94 | A_RES | Resolution | – | – | 12 | bits | – |
| SID95 | A_CHNLS_S | Number of channels - single ended | – | – | 8 | – | – |
| SID96 | A-CHNKS_D | Number of channels - differential | – | – | 4 | – | Diff inputs use neighboring I/O |
| SID97 | A-MONO | Monotonicity | – | – | – | – | Yes |
| SID98 | A_GAINERR | Gain error | – | – | ±0.1 | % | With external reference. |
| SID99 | A_OFFSET | Input offset voltage | – | – | 2 | mV | Measured with 1-V reference –40°C £ TA £ 125°C |
| SID99A | A_OFFSET | Input offset voltage | – | – | 2.7 | mV | 125°C < TA £ 150°C |
| SID100 | A_ISAR | Current consumption | – | – | 1 | mA | – |
| SID101 | A_VINS | Input voltage range - single ended | VSS | – | VDDA | V | – |
| SID102 | A_VIND | Input voltage range - differential | VSS | – | VDDA | V | – |
| SID103 | A_INRES | Input resistance | – | – | 2.2 | KΩ | – |
| SID104 | A_INCAP | Input capacitance | – | – | 10 | pF | – |
| SID260 | VREFSAR | Trimmed internal reference to SAR | – | – | TBD | V | – |

Datasheet	34	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Electrical specifications

Table 16	SAR ADC AC specifications

| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SID106 | A_PSRR | Power supply rejection ratio | 70 | – | – | dB | – |
| SID107 | A_CMRR | Common mode rejection ratio | 66 | – | – | dB | Measured at 1 V, –40°C £ TA £ 125°C |
| SID107A | A_CMRR | Common mode rejection ratio | – | 66 | – | dB | 125°C < TA £ 150°C |
| SID108 | A_SAMP | Sample rate | – | – | 1 | Msps | –40°C £ TA £ 125°C |
| SID108A | A_SAMP | Sample rate | – | – | 0.375 | Msps | 125°C < TA £ 150°C |
| SID109 | A_SNR | Signal-to-noise and distortion ratio (SINAD) | 65 | – | – | dB | FIN = 10 kHz, –40°C £ TA £ 125°C |
| SID109A | A_SNR | Signal-to-noise and distortion ratio (SINAD) | – | 65 | – | dB | 125°C £ TA £ 150°C |
| SID110 | A_BW | Input bandwidth without aliasing | – | – | A_samp/2 | kHz | – |
| SID111 | A_INL | Integral non linearity. VDD = 1.71 V to 5.5 V, 1 Msps | –1.7 | – | 2 | LSB | VREF = 1 V to VDD, –40°C £ TA £ 125°C |
| SID111A | A_INL | Integral non linearity. VDD = 1.71 V to 3.6 V, 1 Msps | –1.5 | – | 1.7 | LSB | VREF = 1.71 V to VDD, –40°C £ TA £ 125°C |
| SID111B | A_INL | Integral non linearity. VDD = 1.71 V to 5.5 V, 500 ksps | –1.5 | – | 1.7 | LSB | VREF = 1 V to VDD, –40°C £ TA £ 125°C |
| SID111C | A_INL | Integral non linearity. VDD = 4.5 V to 5.5 V, 375 ksps | –4.5 | – | 3.3 | LSB | VREF = 1 V to VDD, 125°C < TA £ 150°C |
| SID111D | A_INL | Integral non linearity. VDD = 3 V to 4.5 V, 300 ksps | –4.5 | – | 3.3 | LSB | VREF = 1 V to VDD, 125°C < TA £ 150°C |
| SID111E | A_INL | Integral non linearity. VDD = 1.71 V to 3 V, 150 ksps | –4.5 | – | 3.4 | LSB | VREF = 1 V to VDD, 125°C < TA £ 150°C |
| SID112 | A_DNL | Differential non linearity. VDD = 1.71 V to 5.5 V, 1 Msps | –1 | – | 2.2 | LSB | VREF = 1 V to VDD, –40°C £ TA £ 125°C |
| SID112A | A_DNL | Differential non linearity. VDD = 1.71 V to 3.6 V, 1 Msps | –1 | – | 2 | LSB | VREF = 1.71 V to VDD, –40°C £ TA £ 125°C |
| SID112B | A_DNL | Differential non linearity. VDD = 1.71 V to 5.5 V, 500 ksps | –1 | – | 2.2 | LSB | VREF = 1 V to VDD, –40°C £ TA £ 125°C |
| SID112C | A_DNL | Differential non linearity. VDD = 4.5 V to 5.5 V, 375 ksps | –1 | – | 3.2 | LSB | VREF = 1 V to VDD, 125°C < TA £ 150°C |
| SID112D | A_DNL | Differential non linearity. VDD = 3 V to 4.5 V, 300 ksps | –1 | – | 3.2 | LSB | VREF = 1 V to VDD, 125°C < TA £ 150°C |
| SID112E | A_DNL | Differential non linearity. VDD = 1.71 V to 3 V, 150 ksps | –1 | – | 3.3 | LSB | VREF = 1 V to VDD, 125°C < TA £ 150°C |
| SID113 | A_THD | Total harmonic distortion | – | – | –65 | dB | Fin = 10 kHz |
| SID113A | A_THD | Total harmonic distortion | – | –65 | – | dB | Fin = 10 kHz, 125°C < TA £ 150°C |
| SID261 | FSARINTREF | SAR operating speed without external reference bypass | – | – | 100 | ksps | 12-bit resolution |

Datasheet	35	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Electrical specifications

5.3.5	CSD and IDAC

Table 17	CSD and IDAC specifications

| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SYS.PER#3 | VDD_RIPPLE | Max allowed ripple on power supply, DC to 10 MHz | – | – | ±50 | mV | VDD > 2 V (with ripple), 25°C TA, Sensitivity = 0.1 pF |
| SYS.PER#16 | VDD_RIPPLE_1.8 | Max allowed ripple on power supply, DC to 10 MHz | – | – | ±25 | mV | VDD > 1.75 V (with ripple), 25°C TA, Parasitic Capacitance (CP) < 20 pF, Sensitivity ≥ 0.4 pF |
| SID.CSD.BLK | ICSD | Maximum block current | – | – | 4000 | µA | Maximum block current for both IDACs in dynamic (switching) mode including comparators, buffer, and reference generator. |
| SID.CSD#15 | VREF | Voltage reference for CSD and Comparator | 0.6 | 1.2 | VDDA – 0.6 | V | VDDA – 0.06 or 4.4, whichever is lower |
| SID.CSD#15A | VREF_EXT | External Voltage reference for CSD and Comparator | 0.6 |  | VDDA – 0.6 |  | VDDA – 0.06 or 4.4, whichever is lower |
| SID.CSD#16 | IDAC1IDD | IDAC1 (7 bits) block current | – | – | 1750 | µA | – |
| SID.CSD#17 | IDAC2IDD | IDAC2 (7 bits) block current | – | – | 1750 |  | – |
| SID308 | VCSD | Voltage range of operation | 1.71 | – | 5.5 | V | 1.8 V ± 5% or 1.8 V to 5.5 V |
| SID308A | VCOMPIDAC | Voltage compliance range of IDAC | 0.6 | – | VDDA – 0.6 |  | VDDA – 0.06 or 4.4, whichever is lower |
| SID309 | IDAC1DNL | DNL | –1 | – | 1 | LSB |  |
| SID310 | IDAC1INL | INL | –3 | – | 3 |  | INL is ±5 LSB for VDDA < 2 V |
| SID311 | IDAC2DNL | DNL | –1 | – | 1 |  |  |
| SID312 | IDAC2INL | INL | –3 | – | 3 |  | INL is ±5 LSB for VDDA < 2 V, –40°C £ TA £ 125°C |
| SID312A | IDAC2INL | INL | – | – | 7 |  | 125°C < TA £ 150°C |
| SID313 | SNR | Ratio of counts of finger to noise. Guaranteed by characterization | 5 | – | – | – | Capacitance range of 5 to 35 pF, 0.1-pF sensitivity. All use cases. VDDA > 2 V |

Datasheet	36	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Electrical specifications

Table 17	CSD and IDAC specifications (continued)

| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SID314 | IDAC1CRT1 | Output current of IDAC1 (7 bits) in low range | 4.2 | – | 5.2 | p.A | LSB = 37.5 nA typ |
| SID314A | IDAC1CRT2 | Output current of IDAC1 (7 bits) in medium range | 34 | – | 41 |  | LSB = 300 nA typ |
| SID314B | IDAC1CRT3 | Output current of IDAC1 (7 bits) in high range | 275 | – | 330 |  | LSB = 2.4 p.A typ |
| SID314C | IDAC1CRT12 | Output current of IDAC1 (7 bits) in low range, 2X mode | 8 | – | 10.5 |  | LSB = 37.5 nA typ. 2X output stage |
| SID314D | IDAC1CRT22 | Output current of IDAC1 (7 bits) in medium range, 2X mode | 69 | – | 82 |  | LSB = 300 nA typ. 2X output stage |
| SID314E | IDAC1CRT32 | Output current of IDAC1 (7 bits) in high range, 2X mode | 540 | – | 660 |  | LSB = 2.4 p.A typ 2X output stage |
| SID315 | IDAC2CRT1 | Output current of IDAC2 (7 bits) in low range | 4.2 | – | 5.2 |  | LSB = 37.5 nA typ |
| SID315A | IDAC2CRT2 | Output current of IDAC2 (7 bits) in medium range | 34 | – | 41 | p.A | LSB = 300 nA typ |
| SID315B | IDAC2CRT3 | Output current of IDAC2 (7 bits) in high range | 275 | – | 330 |  | LSB = 2.4 p.A typ |
| SID315C | IDAC2CRT12 | Output current of IDAC2 (7 bits) in low range, 2X mode | 8 | – | 10.5 |  | LSB = 37.5 nA typ. 2X output stage |
| SID315D | IDAC2CRT22 | Output current of IDAC2 (7 bits) in medium range, 2X mode | 69 | – | 82 |  | LSB = 300 nA typ. 2X output stage |
| SID315E | IDAC2CRT32 | Output current of IDAC2 (7 bits) in high range, 2X mode | 540 | – | 660 |  | LSB = 2.4 p.A typ 2X output stage |
| SID315F | IDAC3CRT13 | Output current of IDAC in 8-bit mode in low range | 8 | – | 10.5 |  | LSB = 37.5 nA typ |
| SID315G | IDAC3CRT23 | Output current of IDAC in 8-bit mode in medium range | 69 | – | 82 |  | LSB = 300 nA typ |
| SID315H | IDAC3CRT33 | Output current of IDAC in 8-bit mode in high range | 540 | – | 660 |  | LSB = 2.4 p.A typ |
| SID320 | IDACOFFSET | All zeros input | – | – | 1 | LSB | Polarity set by Source or Sink. Offset is 2 LSBs for 37.5 nA/LSB mode |

Datasheet	37	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Electrical specifications

Table 17	CSD and IDAC specifications (continued)

| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SID321 | IDACGAIN | Full-scale error less offset | – | – | ±10 | % | –40°C £ TA £ 125°C |
| SID321A | IDACGAIN | Full-scale error less offset | – | – | ±11 |  | 125°C < TA £ 150°C |
| SID322 | IDACMISMATCH1 | Mismatch between IDAC1 and IDAC2 in Low mode | – | – | 9.2 | LSB | LSB = 37.5 nA typ |
| SID322A | IDACMISMATCH2 | Mismatch between IDAC1 and IDAC2 in Medium mode | – | – | 4.6 |  | LSB = 300 nA typ |
| SID322B | IDACMISMATCH3 | Mismatch between IDAC1 and IDAC2 in High mode | – | – | 2.3 |  | LSB = 2.4 µA typ, –40°C £ TA £ 125°C |
| SID322C | IDACMISMATCH4 | Mismatch between IDAC1 and IDAC2 in High mode | – | – | 6.3 |  | 125°C < TA £ 150°C |
| SID323 | IDACSET8 | Settling time to 0.5 LSB for 8-bit IDAC | – | – | 10 | µs | Full-scale transition. No external load. |
| SID324 | IDACSET7 | Settling time to 0.5 LSB for 7-bit IDAC | – | – | 10 |  | Full-scale transition. No external load. |
| SID325 | CMOD | External modulator capacitor | – | 2.2 | – | nF | 6-V rating, X7R cap, or NP0/C0G cap for hot temperature operation. |

Datasheet	38	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Electrical specifications

5.3.6	10-bit CAPSENSETM ADC

Table 18	10-bit CAPSENSETM ADC specifications

| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SIDA94 | A_RES | Resolution | – | – | 10 | bits | Auto-zeroing is required every millisecond. |
| SIDA95 | A_CHNLS_S | Number of channels - single ended | – | – | 16 |  | Defined by AMUX bus. |
| SIDA97 | A-MONO | Monotonicity | – | – | – | Yes | – |
| SIDA98 | A_GAINERR | Gain error | – | – | ±2 | % | In VREF (2.4 V) mode with VDDA bypass capacitance of 10 µF |
| SIDA99 | A_OFFSET | Input offset voltage | – | – | 3 | mV | In VREF (2.4 V) mode with VDDA bypass capacitance of 10 µF |
| SIDA100 | A_ISAR | Current consumption | – | – | 0.25 | mA | – |
| SIDA101 | A_VINS | Input voltage range - single ended | VSSA | – | VDDA | V | – |
| SIDA103 | A_INRES | Input resistance | – | 2.2 | – | KΩ | – |
| SIDA104 | A_INCAP | Input capacitance | – | 20 | – | pF | – |
| SIDA106 | A_PSRR | Power supply rejection ratio | – | 60 | – | dB | In VREF (2.4 V) mode with VDDA bypass capacitance of 10 µF |
| SIDA107 | A_TACQ | Sample acquisition time | – | 1 | – | µs | – |
| SIDA108 | A_CONV8 | Conversion time for 8-bit resolution at conversion rate = Fhclk/(2"(N + 2)). Clock frequency = 48 MHz. | – | – | 21.3 | µs | Does not include acquisition time. Equivalent to 44.8 ksps including acquisition time. |
| SIDA108A | A_CONV10 | Conversion time for 10-bit resolution at conversion rate = Fhclk/(2"(N + 2)). Clock frequency = 48 MHz. | – | – | 85.3 | µs | Does not include acquisition time. Equivalent to 11.6 ksps including acquisition time. |
| SIDA109 | A_SND | Signal-to-noise and Distortion ratio (SINAD) | 61 | – | – | dB | With 10-Hz input sine wave, external 2.4-V reference, VREF (2.4 V) mode |
| SIDA110 | A_BW | Input bandwidth without aliasing | – | – | 22.4 | kHz | 8-bit resolution |
| SIDA111 | A_INL | Integral Non Linearity. 1 ksps | – | – | 2 | LSB | VREF = 2.4 V or greater |
| SIDA112 | A_DNL | Differential Non Linearity. 1 ksps | – | – | 1 | LSB | – |

Datasheet	39	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Electrical specifications

5.4	Digital peripherals

5.4.1	Timer counter pulse-width modulator (TCPWM)

Table 19	TCPWM specifications

| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SID.TCPWM.1 | ITCPWM1 | Block current consumption at 3 MHz | – | – | 45 | µA | All modes (TCPWM) |
| SID.TCPWM.2 | ITCPWM2 | Block current consumption at 12 MHz | – | – | 155 |  | All modes (TCPWM) |
| SID.TCPWM.2A | ITCPWM3 | Block current consumption at 48 MHz | – | – | 650 |  | All modes (TCPWM) |
| SID.TCPWM.3 | TCPWMFREQ | Operating frequency | – | – | Fc | MHz | Fc max = CLK_SYS Maximum = 48 MHz |
| SID.TCPWM.4 | TPWMENEXT | Input trigger pulse width | 2/Fc | – | – | ns | For all trigger events[8] |
| SID.TCPWM.5 | TPWMEXT | Output trigger pulse widths | 2/Fc | – | – |  | Minimum possible width of Overflow, Underflow, and CC (Counter equals Compare value) outputs |
| SID.TCPWM.5A | TCRES | Resolution of counter | 1/Fc | – | – |  | Minimum time between successive counts |
| SID.TCPWM.5B | PWMRES | PWM resolution | 1/Fc | – | – |  | Minimum pulse width of PWM Output |
| SID.TCPWM.5C | QRES | Quadrature inputs resolution | 1/Fc | – | – |  | Minimum pulse width between Quadrature phase inputs |

Note

8. Trigger events can be Stop, Start, Reload, Count, Capture, or Kill depending on which mode of operation is

selected.

Datasheet	40	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Electrical specifications

5.4.2	I2C

Table 20	Fixed I2C DC specifications[9]

| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SID149 | II2C1 | Block current consumption at 100 kHz | – | – | 50 | µA | – |
| SID150 | II2C2 | Block current consumption at 400 kHz | – | – | 135 |  | – |
| SID151 | II2C3 | Block current consumption at 1 Mbps | – | – | 310 |  | – |
| SID152 | II2C4 | I2C enabled in Deep Sleep mode | – | – | 1.4 |  | – |

| Table 21 Fixed I2C AC specifications[9] |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| SID153 | FI2C1 | Bit rate | – | – | 1 | Msps | – |

5.4.3	SPI

Table 22	SPI DC specifications[9]

| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SID163 | ISPI1 | Block current consumption at 1 Mbps | – | – | 360 | µA | – |
| SID164 | ISPI2 | Block current consumption at 4 Mbps | – | – | 560 |  | – |
| SID165 | ISPI3 | Block current consumption at 8 Mbps | – | – | 600 |  | – |

Table 23	SPI AC specifications[9]

| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SID166 | FSPI | SPI Operating frequency (master; 6X oversampling) | – | – | 8 | MHz | – |

Fixed SPI Master Mode AC Specifications

| SID167 | TDMO | MOSI Valid after SClock driving edge | – | – | 15 | ns | – |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SID168 | TDSI | MISO Valid before SClock capturing edge | 20 | – | – |  | Full clock, late MISO sampling |
| SID169 | THMO | Previous MOSI data hold time | 0 | – | – |  | Referred to Slave capturing edge |

Note

9. Guaranteed by characterization.

Datasheet	41	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |
| Electrical specifications Table 23 SPI AC specifications[9] (continued) |  |

| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |

Fixed SPI Slave Mode AC Specifications

| SID170 | TDMI | MOSI Valid before Sclock Capturing edge | 40 | – | – | ns | – |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SID171 | TDSO | MISO Valid after Sclock driving edge | – | – | 42 + (3 × Tcpu) |  | TCPU = 1/FCPU |
| SID171A | TDSO_EXT | MISO Valid after Sclock driving edge in Ext. Clk mode | – | – | 48 |  | – |
| SID172 | THSO | Previous MISO data hold time | 0 | – | – |  | – |
| SID172A | TSSELSSCK | SSEL Valid to first SCK valid edge | – | – | 100 | ns | – |

5.4.4	UART

Table 24	UART DC specifications[10]

| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SID160 | IUART1 | Block current consumption at 100 Kbps | – | – | 55 | µA | – |
| SID161 | IUART2 | Block current consumption at 1000 Kbps | – | – | 312 | µA | – |

| Table 25 UART AC specifications[10] |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| SID162 | FUART | Bit rate | – | – | 1 | Mbps | – |

5.4.5	LCD

Table 26	LCD Direct Drive DC specifications[10]

| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SID154 | ILCDLOW | Operating current in low power mode | – | 5 | – | µA | 16 x 4 small segment disp. at 50 Hz |
| SID155 | CLCDCAP | LCD capacitance per segment/common driver | – | 500 | 5000 | pF | – |
| SID156 | LCDOFFSET | Long-term segment offset | – | 20 | – | mV | – |
| SID157 | ILCDOP1 | LCD system operating current Vbias = 5 V | – | 2 | – | mA | 32 x 4 segments. 50 Hz, 25°C |
| SID158 | ILCDOP2 | LCD system operating current Vbias = 3.3 V | – | 2 | – |  | 32 x 4 segments. 50 Hz, 25°C |

| Table 27 LCD Direct Drive AC specifications[10] |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| SID159 | FLCD | LCD frame rate | 10 | 50 | 150 | Hz | – |

Note

10.Guaranteed by characterization.

Datasheet	42	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Electrical specifications

5.5	Memory

5.5.1	Flash

| Table 28 Flash DC specifications |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| SID173 | VPE | Erase and program voltage | 1.71 | – | 5.5 | V | – |

Table 29	Flash AC specifications

| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SID174 | TROWWRITE[11] | Row (block) write time (erase and program) | – | – | 20 | ms | Row (block) = 128 bytes |
| SID175 | TROWERASE[11] | Row erase time | – | – | 16 |  | – |
| SID176 | TROWPROGRAM[11] | Row program time after erase | – | – | 4 |  | – |
| SID178 | TBULKERASE[11] | Bulk erase time (64 KB) | – | – | 35 |  | – |
| SID180[12] | TDEVPROG[11] | Total device program time | – | – | 7 | seconds | – |
| SID181[12] | FEND | Flash endurance | 100K | – | – | cycles | – |
| SID182[12, 13] | FRET | Flash retention. TA £ 55°C, 100K P/E cycles | 20 | – | – | years | – |
| SID182A[12, 13] | FRET | Flash retention. TA £ 85°C, 10K P/E cycles | 10 | – | – |  | – |
| SID256 | TWS48 | Number of wait states at 48 MHz | 2 | – | – | – | CPU execution from flash |
| SID257 | TWS24 | Number of wait states at 24 MHz | 1 | – | – | – | CPU execution from flash |

Note

11..It can take as much as 20 ms to write to Flash. During this time the device should not be Reset, or Flash operations will be interrupted and cannot be relied on to have completed. Reset sources include the XRES pin, software resets, CPU lockup states and privilege violations, improper power supply levels, and watch-dogs. Make certain that these are not inadvertently activated.

12.Guaranteed by characterization.

13.Infineon provides a retention calculator to calculate the retention lifetime based on the customers' individual temperature profiles for operation over the –40°C to +150°C ambient temperature range. For more information, visit the Infineon community page.

Datasheet	43	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Electrical specifications

5.6	System resources

5.6.1	Power-on reset (POR)

Table 30	Power-on reset (PRES)

| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SID.CLK#6 | SR_POWER | Power supply slew rate | 1[14' | – | 67 | V/ms | On power-up and power-down |
| SID185[15' | VRISEIPOR | Rising trip voltage | 0.80 | – | 1.5 | V | – |
| SID186[15' | VFALLIPOR | Falling trip voltage | 0.70 | – | 1.4 |  | – |

Table 31	Brownout detect (BOD) for VCCD

| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SID190[15' | VFALLPPOR | BOD trip voltage in Active and sleep modes | 1.48 | – | 1.62 | V | – |
| SID192[15' | VFALLDPSLP | BOD trip voltage in Deep Sleep mode | 1.11 | – | 1.5 |  | – |

5.6.2	SWD interface

Table 32	SWD interface specifications

| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SID213 | F_SWDCLK1 | 3.3 V £ VDD £ 5.5 V | – | – | 14 | MHz | SWDCLK ≤ 1/3 CPU clock frequency |
| SID214 | F_SWDCLK2 | 1.71 V £ VDD £ 3.3 V | – | – | 7 |  | SWDCLK ≤ 1/3 CPU clock frequency |
| SID215[15' | T_SWDI_SETUP | T = 1/f SWDCLK | 0.25 × T | – | – | ns | – |
| SID216[15' | T_SWDI_HOLD | T = 1/f SWDCLK | 0.25 × T | – | – |  | – |
| SID217[15' | T_SWDO_VALID | T = 1/f SWDCLK | – | – | 0.5 × T |  | – |
| SID217A[15' | T_SWDO_HOLD | T = 1/f SWDCLK | 1 | – | – |  | – |

Notes

14.If minimum ramp rate cannot be met, XRES should be asserted during voltage ramp (1.5 V > VDDD > 1.0 V for

ramp-down or until voltage is stable for ramp-up). Note that a glitch on the I2C bus could occur during

voltage ramp in this case.

15.Guaranteed by characterization.

Datasheet	44	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Electrical specifications

5.6.3	Internal main oscillator (IMO)

Table 33	IMO DC specifications
(Guaranteed by design)

| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SID218 | IIMO1 | IMO operating current at 48 MHz | – | – | 250 | µA | – |
| SID219 | IIMO2 | IMO operating current at 24 MHz | – | – | 180 | µA | – |

Table 34	IMO AC specifications

| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SID223 | FIMOTOL1 | Frequency variation at 24, 32, and 48 MHz (trimmed) | – | – | ±2 | % | –40°C £ TA £ 125°C |
| SID223A | FIMOTOL1 | Frequency variation at 24, 32, and 48 MHz (trimmed) | – | – | ±4 | % | 125°C £ TA £ 150°C |
| SID226 | TSTARTIMO | IMO startup time | – | – | 7 | µs | – |
| SID228 | TJITRMSIMO2 | RMS jitter at 24 MHz | – | 145 | – | ps | – |

5.6.4	Internal low-speed oscillator (ILO)

Table 35	ILO DC specifications
(Guaranteed by design)

| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SID231[16' | IILO1 | ILO operating current | – | 0.3 | 1.05 | µA | – |

Table 36	ILO AC specifications

| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SID234[16' | TSTARTILO1 | ILO startup time | – | – | 2 | ms | – |
| SID236[16' | TILODUTY | ILO duty cycle | 40 | 50 | 60 | % | – |
| SID237 | FILOTRIM1 | ILO frequency range | 20 | 40 | 80 | kHz | – |

Note

16.Guaranteed by characterization.

Datasheet	45	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Electrical specifications

5.6.5	Watch crystal oscillator

Table 37	Watch crystal oscillator specifications

| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SID398 | FWCO | Crystal frequency | – | 32.768 | – | kHz | – |
| SID399 | FTOL | Frequency tolerance | – | 50 | 250 | ppm | With 20-ppm crystal |
| SID400 | ESR | Equivalent series resistance | – | 50 | – | kΩ | – |
| SID401 | PD | Drive Level | – | – | 1 | µW | – |
| SID402 | TSTART | Startup time | – | – | 500 | ms | – |
| SID403 | CL | Crystal load capacitance | 6 | – | 12.5 | pF | – |
| SID404 | C0 | Crystal shunt capacitance | – | 1.35 | – | pF | – |
| SID405 | IWCO1 | Operating current (high power mode) | – | – | 8 | µA | – |
| SID406 | IWCO2 | Operating current (low power mode) | – | – | 1 | µA | – |

5.6.6	External clock

Table 38	External clock specifications

| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SID305[17' | ExtClkFreq | External clock input frequency | 0 | – | 48 | MHz | – |
| SID306[17' | ExtClkDuty | Duty cycle; measured at VDD/2 | 45 | – | 55 | % | – |

5.6.7	Block

Table 39	Block specs

| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SID262[17' | TCLKSWITCH | System clock source switching time | 3 | – | 4 | Periods | – |

5.6.8	PRGIO pass-through time

Table 40	PRGIO pass-through time (Delay in bypass mode)

| Spec ID | Parameter | Description | Min | Typ | Max | Unit | Details/conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SID252 | PRG_BYPASS | Max. delay added by PRGIO in bypass mode | – | – | 1.6 | ns | – |

Note

17.Guaranteed by characterization.

Datasheet	46	002-15106 Rev. *L

2025-06-06

| Ordering information | PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |
| --- | --- |

Datasheet	47	002-15106 Rev. *L

2025-06-06

6	Ordering information

Table 41 lists the marketing part numbers for the PSOCTM Automotive 4100S family.

Table 41	Ordering information

| Product | Features |  |  |  |  |  |  |  |  |  |  |  |  | Packages |  |  | Operating temperature |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Max CPU Speed (MHz) | Flash (KB) | SRAM (KB) | UDB | Opamp (CTBm) | CAPSENSETM | Direct LCD Drive | 12-bit SAR ADC | LP Comparators | TCPWM Blocks | SCB Blocks | Smart IOs | GPIO | 28-SSOP |  | 40-QFN | -40°C to +85°C | -40°C to +105°C | -40°C to +125°C |
| CY8C4124PVA-S412 | 24 | 16 | 4 | – | 2 | X | X | – | 2 | 5 | 2 | 16 | 24 | X |  | – | X | – | – |
| CY8C4124LQA-S413 | 24 | 16 | 4 | – | 2 | X | X | – | 2 | 5 | 2 | 16 | 34 | – |  | X | X | – | – |
| CY8C4124PVA-S422 | 24 | 16 | 4 | – | 2 | – | X | 806 Ksps | 2 | 5 | 2 | 16 | 24 | X |  | – | X | – | – |
| CY8C4124LQA-S423 | 24 | 16 | 4 | – | 2 | – | X | 806 Ksps | 2 | 5 | 2 | 16 | 34 | – |  | X | X | – | – |
| CY8C4124PVA-S432 | 24 | 16 | 4 | – | 2 | X | X | 806 Ksps | 2 | 5 | 2 | 16 | 24 | X |  | – | X | – | – |
| CY8C4124LQA-S433 | 24 | 16 | 4 | – | 2 | X | X | 806 Ksps | 2 | 5 | 2 | 16 | 34 | – |  | X | X | – | – |
| CY8C4125PVA-S412 | 24 | 32 | 4 | – | 2 | X | X | – | 2 | 5 | 2 | 16 | 24 | X |  | – | X | – | – |
| CY8C4125LQA-S413 | 24 | 32 | 4 | – | 2 | X | X | – | 2 | 5 | 2 | 16 | 34 | – |  | X | X | – | – |
| CY8C4125PVA-S422 | 24 | 32 | 4 | – | 2 | – | X | 806 Ksps | 2 | 5 | 2 | 16 | 24 | X |  | – | X | – | – |
| CY8C4125LQA-S423 | 24 | 32 | 4 | – | 2 | – | X | 806 Ksps | 2 | 5 | 2 | 16 | 34 | – |  | X | X | – | – |
| CY8C4125PVA-S432 | 24 | 32 | 4 | – | 2 | X | X | 806 Ksps | 2 | 5 | 2 | 16 | 24 | X |  | – | X | – | – |
| CY8C4125LQA-S433 | 24 | 32 | 4 | – | 2 | X | X | 806 Ksps | 2 | 5 | 2 | 16 | 34 | – |  | X | X | – | – |
| CY8C4146PVA-S422 | 48 | 64 | 8 | – | 2 | – | X | 1000 Ksps | 2 | 5 | 2 | 16 | 24 | X |  | – | X | – | – |
| CY8C4146LQA-S423 | 48 | 64 | 8 | – | 2 | – | X | 1000 Ksps | 2 | 5 | 3 | 16 | 34 | – |  | X | X | – | – |
| CY8C4146PVA-S432 | 48 | 64 | 8 | – | 2 | X | X | 1000 Ksps | 2 | 5 | 3 | 16 | 24 | X |  | – | X | – | – |

| Ordering information | PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |
| --- | --- |

Datasheet	48	002-15106 Rev. *L

2025-06-06

Table 41	Ordering information (continued)

| Product | Features |  |  |  |  |  |  |  |  |  |  |  |  | Packages |  |  | Operating temperature |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Max CPU Speed (MHz) | Flash (KB) | SRAM (KB) | UDB | Opamp (CTBm) | CAPSENSETM | Direct LCD Drive | 12-bit SAR ADC | LP Comparators | TCPWM Blocks | SCB Blocks | Smart IOs | GPIO | 28-SSOP |  | 40-QFN | -40°C to +85°C | -40°C to +105°C | -40°C to +125°C |
| CY8C4146LQA-S433 | 48 | 64 | 8 | – | 2 | X | X | 1000 Ksps | 2 | 5 | 3 | 16 | 34 | – |  | X | X | – | – |
| CY8C4124PVS-S412 | 24 | 16 | 4 | – | 2 | X | X | – | 2 | 5 | 2 | 16 | 24 | X |  | – | – | X | – |
| CY8C4124LQS-S413 | 24 | 16 | 4 | – | 2 | X | X | – | 2 | 5 | 2 | 16 | 34 | – |  | X | – | X | – |
| CY8C4124PVS-S422 | 24 | 16 | 4 | – | 2 | – | X | 806 Ksps | 2 | 5 | 2 | 16 | 24 | X |  | – | – | X | – |
| CY8C4124LQS-S423 | 24 | 16 | 4 | – | 2 | – | X | 806 Ksps | 2 | 5 | 2 | 16 | 34 | – |  | X | – | X | – |
| CY8C4124PVS-S432 | 24 | 16 | 4 | – | 2 | X | X | 806 Ksps | 2 | 5 | 2 | 16 | 24 | X |  | – | – | X | – |
| CY8C4124LQS-S433 | 24 | 16 | 4 | – | 2 | X | X | 806 Ksps | 2 | 5 | 2 | 16 | 34 | – |  | X | – | X | – |
| CY8C4125PVS-S412 | 24 | 32 | 4 | – | 2 | X | X | – | 2 | 5 | 2 | 16 | 24 | X |  | – | – | X | – |
| CY8C4125LQS-S413 | 24 | 32 | 4 | – | 2 | X | X | – | 2 | 5 | 2 | 16 | 34 | – |  | X | – | X | – |
| CY8C4125PVS-S422 | 24 | 32 | 4 | – | 2 | – | X | 806 Ksps | 2 | 5 | 2 | 16 | 24 | X |  | – | – | X | – |
| CY8C4125LQS-S423 | 24 | 32 | 4 | – | 2 | – | X | 806 Ksps | 2 | 5 | 2 | 16 | 34 | – |  | X | – | X | – |
| CY8C4125PVS-S432 | 24 | 32 | 4 | – | 2 | X | X | 806 Ksps | 2 | 5 | 2 | 16 | 24 | X |  | – | – | X | – |
| CY8C4125LQS-S433 | 24 | 32 | 4 | – | 2 | X | X | 806 Ksps | 2 | 5 | 2 | 16 | 34 | – |  | X | – | X | – |
| CY8C4146PVS-S422 | 48 | 64 | 8 | – | 2 | – | X | 1000 Ksps | 2 | 5 | 2 | 16 | 24 | X |  | – | – | X | – |
| CY8C4146LQS-S423 | 48 | 64 | 8 | – | 2 | – | X | 1000 Ksps | 2 | 5 | 3 | 16 | 34 | – |  | X | – | X | – |
| CY8C4146PVS-S432 | 48 | 64 | 8 | – | 2 | X | X | 1000 Ksps | 2 | 5 | 3 | 16 | 24 | X |  | – | – | X | – |
| CY8C4146LQS-S433 | 48 | 64 | 8 | – | 2 | X | X | 1000 Ksps | 2 | 5 | 3 | 16 | 34 | – |  | X | – | X | – |
| CY8C4124PVE-S412 | 24 | 16 | 4 | – | 2 | X | X | – | 2 | 5 | 2 | 16 | 24 | X |  | – | – | – | X |
| CY8C4124LQE-S413 | 24 | 16 | 4 | – | 2 | X | X | – | 2 | 5 | 2 | 16 | 34 | – |  | X | – | – | X |

| Ordering information | PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |
| --- | --- |

Datasheet	49	002-15106 Rev. *L

2025-06-06

Table 41	Ordering information (continued)

| Product | Features |  |  |  |  |  |  |  |  |  |  |  |  | Packages |  |  | Operating temperature |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Max CPU Speed (MHz) | Flash (KB) | SRAM (KB) | UDB | Opamp (CTBm) | CAPSENSETM | Direct LCD Drive | 12-bit SAR ADC | LP Comparators | TCPWM Blocks | SCB Blocks | Smart IOs | GPIO | 28-SSOP |  | 40-QFN | -40°C to +85°C | -40°C to +105°C | -40°C to +125°C |
| CY8C4124PVE-S422 | 24 | 16 | 4 | – | 2 | – | X | 806 Ksps | 2 | 5 | 2 | 16 | 24 | X |  | – | – | – | X |
| CY8C4124LQE-S423 | 24 | 16 | 4 | – | 2 | – | X | 806 Ksps | 2 | 5 | 2 | 16 | 34 | – |  | X | – | – | X |
| CY8C4124PVE-S432 | 24 | 16 | 4 | – | 2 | X | X | 806 Ksps | 2 | 5 | 2 | 16 | 24 | X |  | – | – | – | X |
| CY8C4124LQE-S433 | 24 | 16 | 4 | – | 2 | X | X | 806 Ksps | 2 | 5 | 2 | 16 | 34 | – |  | X | – | – | X |
| CY8C4125PVE-S412 | 24 | 32 | 4 | – | 2 | X | X | – | 2 | 5 | 2 | 16 | 24 | X |  | – | – | – | X |
| CY8C4125LQE-S413 | 24 | 32 | 4 | – | 2 | X | X | – | 2 | 5 | 2 | 16 | 34 | – |  | X | – | – | X |
| CY8C4125PVE-S422 | 24 | 32 | 4 | – | 2 | – | X | 806 Ksps | 2 | 5 | 2 | 16 | 24 | X |  | – | – | – | X |
| CY8C4125LQE-S423 | 24 | 32 | 4 | – | 2 | – | X | 806 Ksps | 2 | 5 | 2 | 16 | 34 | – |  | X | – | – | X |
| CY8C4125PVE-S432 | 24 | 32 | 4 | – | 2 | X | X | 806 Ksps | 2 | 5 | 2 | 16 | 24 | X |  | – | – | – | X |
| CY8C4125LQE-S433 | 24 | 32 | 4 | – | 2 | X | X | 806 Ksps | 2 | 5 | 2 | 16 | 34 | – |  | X | – | – | X |
| CY8C4146PVE-S422 | 48 | 64 | 8 | – | 2 | – | X | 1000 Ksps | 2 | 5 | 2 | 16 | 24 | X |  | – | – | – | X |
| CY8C4146LQE-S423 | 48 | 64 | 8 | – | 2 | – | X | 1000 Ksps | 2 | 5 | 3 | 16 | 34 | – |  | X | – | – | X |
| CY8C4146PVE-S432 | 48 | 64 | 8 | – | 2 | X | X | 1000 Ksps | 2 | 5 | 3 | 16 | 24 | X |  | – | – | – | X |
| CY8C4146LQE-S433 | 48 | 64 | 8 | – | 2 | X | X | 1000 Ksps | 2 | 5 | 3 | 16 | 34 | – |  | X | – | – | X |

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Ordering information

The nomenclature used in the preceding table is based on the following part numbering convention: Table 42	Nomenclature

| Field | Description | Values | Meaning |
| --- | --- | --- | --- |
| CY | Cypress (an Infineon company) Prefix | – | – |
| 8C | PSOCTM | – | – |
| 4 | Architecture | 4 | PSOCTM 4 (Arm® Cortex® M0/M0+ based) |
| A | Family | 0 | PSOCTM 4000 family |
|  |  | 1 | PSOCTM 4100 family |
|  |  | 2 | PSOCTM 4200 family |
| B | CPU-speed | 1 | 16 MHz |
|  |  | 2 | 24 MHz |
|  |  | 4 | 48 MHz |
| C | Flash capacity | 4 | 16 KB |
|  |  | 5 | 32 KB |
|  |  | 6 | 64 KB |
| DE | Package code | LQ | QFN |
|  |  | PV | SSOP |
| F | Temperature range | A | Automotive (AEC-Q100: –40°C to +85°C) |
|  |  | S | Automotive (AEC-Q100: –40°C to +105°C) |
|  |  | E | Automotive (AEC-Q100: –40°C to +125°C) |
| S | Silicon family | S | PSOCTM 4A-S1, PSOCTM 4A-S2 |
|  |  | M | PSOCTM 4A-M |
| XYZ | Attributes code | 000–999 | Code of feature set in the specific family |

Datasheet	50	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Ordering information

The following is an example of a part number:

CY 8C 4 A B C D F G – XYZ X T = Tape and reel Attributes code Silicon family Temperature range Package code Flash capacity CPU speed Family within architecture Architecture PSOCTM CYPRESS (an Infineon company) prefix For example, CY8C4124PVE-S422 4: PSOCTM 4 Architecture - PSOCTM 4100 Family - 24 MHz 4: 16 KB PV: SSOP E: Automotive (AEC-Q100: -40°C to +125°C) S: PSOCTM 4 S-series 422: Code feature

Datasheet	51	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Packaging information for PSOCTM Automotive 4100S

7	Packaging information for PSOCTM Automotive 4100S

The Infineon’s 32-bit microcontroller, PSOCTM Automotive 4100S will be offered in 40-pin QFN, and 28-pin SSOP packages.

Table 43 provides the package dimensions and Infineon drawing numbers.

Table 43	Package list

| Spec ID | Package | Description | Package drawing |
| --- | --- | --- | --- |
| BID27 | 40-pin QFN | 6 × 6 × 0.6 mm LD40A 4.6 × 4.6 mm E-Pad (Sawn, Wettable flank package) | 002-16818 |
| BID28 | 28-pin SSOP | 10.2 × 5.3 × 2.0 mm SP28 | 51-85079 |

Table 44	Package thermal characteristics

| Parameter | Description | Package | Conditions | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TA | Operating ambient temperature | – | For A-grade devices | –40 | 25 | 85 | °C |
|  |  | – | For S-grade devices | –40 | 25 | 105 | °C |
|  |  | – | For E-grade devices | –40 | 25 | 125 | °C |
| TJ | Operating junction temperature | – | For A-grade devices | –40 | – | 100 | °C |
|  |  | – | For S-grade devices | –40 | – | 120 | °C |
|  |  | – | For E-grade devices | –40 | – | 140 | °C |
| TJA | Package θJA | 40-pin QFN | – | – | 25 | – | °C/W |
| TJC | Package θJC | 40-pin QFN | – | – | 3 | – | °C/W |
| TJA | Package θJA | 28-pin SSOP | – | – | 66.58 | – | °C/W |
| TJC | Package θJC | 28-pin SSOP | – | – | 46.28 | – | °C/W |

Table 45	Solder reflow peak temperature

| Package All | Maximum peak temperature 260°C | Maximum time at peak temperature 30 seconds |
| --- | --- | --- |

Table 46	Package moisture sensitivity level (MSL), IPC/JEDEC J-STD-020

Package MSL All MSL 3

Datasheet	52	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Packaging information for PSOCTM Automotive 4100S

7.1	Package diagrams

002-16818 Rev. *B

Figure 8	40-pin QFN ((6.0 × 6.0 × 0.6 mm) LD40A 4.6 × 4.6 mm E-Pad (Sawn, Wettable flank package))
package outline (PG-VQFN-40)

Datasheet	53	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Packaging information for PSOCTM Automotive 4100S

51-85079 Rev. *G

Figure 9	28-pin SSOP (10.2 × 5.3 × 2.0 mm) SP28 package outline (PG-SSOP-28)

Datasheet	54	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Acronyms

8	Acronyms

Table 47	Acronyms used in this document

| Acronym | Description |
| --- | --- |
| abus | analog local bus |
| ADC | analog-to-digital converter |
| AG | analog global |
| AHB | AMBA (advanced microcontroller bus architecture) high-performance bus, an Arm data transfer bus |
| ALU | arithmetic logic unit |
| AMUXBUS | analog multiplexer bus |
| API | application programming interface |
| APSR | application program status register |
| Arm® | advanced RISC machine, a CPU architecture |
| ATM | automatic thump mode |
| BW | bandwidth |
| CMRR | common-mode rejection ratio |
| CPU | central processing unit |
| CRC | cyclic redundancy check, an error-checking protocol |
| DAC | digital-to-analog converter, see also IDAC, VDAC |
| DIO | digital input/output, GPIO with only digital capabilities, no analog. See GPIO. |
| DMIPS | Dhrystone million instructions per second |
| DMA | direct memory access, see also TD |
| DNL | differential nonlinearity, see also INL |
| DNU | do not use |
| DR | port write data registers |
| DSI | digital system interconnect |
| DWT | data watchpoint and trace |
| ECC | error correcting code |
| ECO | external crystal oscillator |
| EEPROM | electrically erasable programmable read-only memory |
| EMI | electromagnetic interference |
| EMIF | external memory interface |
| EOC | end of conversion |
| EOF | end of frame |
| EPSR | execution program status register |
| ESD | electrostatic discharge |
| ETM | embedded trace macrocell |
| FPB | flash patch and breakpoint |
| GPIO | general-purpose input/output, applies to a PSOCTM pin |
| HVI | high-voltage interrupt, see also LVI, LVD |
| IC | integrated circuit |

Datasheet	55	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Acronyms

Table 47	Acronyms used in this document (continued)

| Acronym | Description |
| --- | --- |
| IDAC | current DAC, see also DAC, VDAC |
| IDE | integrated development environment |
| I2C, or IIC | Inter-Integrated Circuit, a communications protocol |
| ILO | internal low-speed oscillator, see also IMO |
| IMO | internal main oscillator, see also ILO |
| INL | integral nonlinearity, see also DNL |
| I/O | input/output, see also GPIO, DIO, SIO, USBIO |
| IPOR | initial power-on reset |
| IPSR | interrupt program status register |
| IRQ | interrupt request |
| ITM | instrumentation trace macrocell |
| LCD | liquid crystal display |
| LIN | Local Interconnect Network, a communications protocol. |
| LR | link register |
| LUT | lookup table |
| LVD | low-voltage detect, see also LVI |
| LVI | low-voltage interrupt, see also HVI |
| LVTTL | low-voltage transistor-transistor logic |
| MAC | multiply-accumulate |
| MCU | microcontroller unit |
| MISO | master-in slave-out |
| NC | no connect |
| NMI | nonmaskable interrupt |
| NRZ | non-return-to-zero |
| NVIC | nested vectored interrupt controller |
| NVL | nonvolatile latch, see also WOL |
| opamp | operational amplifier |
| PC | program counter |
| PCB | printed circuit board |
| PGA | programmable gain amplifier |
| PHUB | peripheral hub |
| PHY | physical layer |
| PICU | port interrupt control unit |
| PLL | phase-locked loop |
| PMDD | package material declaration data sheet |
| POR | power-on reset |
| PRES | precise power-on reset |
| PRS | pseudo random sequence |
| PS | port read data register |
| PSOCTM | Programmable System-on-Chip |
| PSRR | power supply rejection ratio |

Datasheet	56	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Acronyms

Table 47	Acronyms used in this document (continued)

| Acronym | Description |
| --- | --- |
| PWM | pulse-width modulator |
| RAM | random-access memory |
| RISC | reduced-instruction-set computing |
| RMS | root-mean-square |
| RTC | real-time clock |
| RTL | register transfer language |
| RTR | remote transmission request |
| RX | receive |
| SAR | successive approximation register |
| SC/CT | switched capacitor/continuous time |
| SCL | I2C serial clock |
| SDA | I2C serial data |
| S/H | sample and hold |
| SINAD | signal to noise and distortion ratio |
| SIO | special input/output, GPIO with advanced features. See GPIO. |
| SOC | start of conversion |
| SOF | start of frame |
| SPI | Serial Peripheral Interface, a communications protocol |
| SR | slew rate |
| SRAM | static random access memory |
| SRES | software reset |
| SWD | serial wire debug, a test protocol |
| SWV | single-wire viewer |
| TD | transaction descriptor, see also DMA |
| THD | total harmonic distortion |
| TIA | transimpedance amplifier |
| TTL | transistor-transistor logic |
| TX | transmit |
| UART | Universal Asynchronous Receiver-Transmitter, a communications protocol |
| UDB | universal digital block |
| USB | Universal Serial Bus |
| USBIO | USB input/output, PSOCTM pins used to connect to a USB port |
| VDAC | voltage DAC, see also DAC, IDAC |
| WDT | watchdog timer |
| WOL | write once latch, see also NVL |
| WRES | watchdog timer reset |
| XRES | external reset I/O pin |
| XTAL | crystal |

Datasheet	57	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Document conventions

9	Document conventions

9.1	Units of measure

Table 48	Unit of measure

| Symbol | Unit of measure |
| --- | --- |
| °C | degrees Celsius |
| dB | decibel |
| fF | femto farad |
| Hz | hertz |
| KB | 1024 bytes |
| kbps | kilobits per second |
| Khr | kilohour |
| kHz | kilohertz |
| kQ | kilo ohm |
| ksps | kilosamples per second |
| LSB | least significant bit |
| Mbps | megabits per second |
| MHz | megahertz |
| MQ | mega-ohm |
| Msps | megasamples per second |
| µA | microampere |
| µF | microfarad |
| µH | microhenry |
| µs | microsecond |
| µV | microvolt |
| µW | microwatt |
| mA | milliampere |
| ms | millisecond |
| mV | millivolt |
| nA | nanoampere |
| ns | nanosecond |
| nV | nanovolt |
| Q | ohm |
| pF | picofarad |
| ppm | parts per million |
| ps | picosecond |
| s | second |
| sps | samples per second |
| sqrtHz | square root of hertz |
| V | volt |

Datasheet	58	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Revision history

Revision history

| Document revision | Date | Description of changes |
| --- | --- | --- |
| *F | 2019-04-24 | Post to external web. |
| *G | 2020-05-04 | Updated Ordering information: No change in part numbers. Updated “part number convention”. Updated to new template. |
| *H | 2020-09-29 | Updated Electrical specifications: Updated Device-level specifications: Updated description. Updated System resources: Updated Power-on reset (POR): Updated Table 30. Refer to Product Information Notice #6965423. |
| *I | 2021-12-01 | Updated Features: Updated description. Updated Functional overview: Updated GPIO: Updated description. Updated Pinouts for 32-bit PSOCTM Automotive 4100S microcontrollers: Updated Table 1. Updated Electrical specifications: Updated Device-level specifications: Updated DC specifications: Updated Table 5. Updated System resources: Updated Power-on reset (POR): Added Note 14 and referred the same note in minimum value of SR_POWER parameter in Table 30. Updated Ordering information: Updated part numbers. Updated Packaging information for PSOCTM Automotive 4100S: Updated Table 43. Updated Table 44. Updated Package diagrams: Added spec 002-18982 Rev. *A. Added spec 002-23807 Rev. *C. Added spec 002-34268 Rev. **. Migrated to Infineon template. |
| *J | 2023-02-21 | Replaced all references of Cypress with Infineon across the document. Updated Ordering information: Updated part numbers. Updated Packaging information for PSOCTM Automotive 4100S: Updated Package diagrams: Removed spec 002-23807 Rev. *C. Removed spec 002-34268 Rev. **. Completing Sunset Review. |

Datasheet	59	002-15106 Rev. *L

2025-06-06

| PSOCTM Automotive 4100S Based on Arm® Cortex®-M0+ CPU |  |
| --- | --- |

Revision history

| Document revision | Date | Description of changes |
| --- | --- | --- |
| *K | 2023-10-10 | Removed 24-pin QFN, 48-pin QFN packages related information in all instances across the document. Updated Pinouts for 32-bit PSOCTM Automotive 4100S microcontrollers: Updated Table 1. Updated Ordering information: Updated part numbers. Removed Note “Alternate fab available.” and its references in Table 41. Updated Packaging information for PSOCTM Automotive 4100S: Updated Table 43. Updated Package diagrams: Removed spec 002-18982 Rev. *A. Updated to new template. |
| *L | 2025-06-06 | Updated title from “Automotive PSOCTM 4: PSOCTM 4100S family” to “PSOCTM Automotive 4100S” Updated PSoCTM to PSOCTM all over the document. Updated Features and Documentation section. Added Function description section. Updated Figure 1, Figure 2, and Figure 3. Added Figure 4 and Figure 5. Updated Table 2, Table 17andTable 43. Added Table 3. Replaced Figure 8 with 002-16818 Rev. *B. |

Datasheet	60	002-15106 Rev. *L

2025-06-06

Trademarks

All referenced product or service names and trademarks are the property of their respective owners.

PSOCTM, formerly known as PSoCTM, is a trademark of Infineon Technologies. Any references to PSoCTM in this document or others shall be deemed to refer

to PSOCTM.

IMPORTANT NOTICE

The information given in this document shall in no event be regarded as a guarantee of conditions or characteristics (“Beschaffenheitsgarantie”).

With respect to any examples, hints or any typical values stated herein and/or any information regarding the application of the product, Infineon Technologies hereby disclaims any and all warranties and liabilities of any kind, including without limitation warranties of non-infringement of intellectual property rights of any third party.

In addition, any information given in this document is subject to customer’s compliance with its obligations stated in this document and any applicable legal requirements, norms and standards concerning customer’s products and any use of the product of Infineon Technologies in customer’s applications.

WARNINGS

Due to technical requirements products may contain dangerous substances. For information on the types in question please contact your nearest Infineon Technologies office.

Except as otherwise explicitly approved by Infineon Technologies in a written document signed by

authorized	representatives	of	Infineon
Technologies, Infineon Technologies’ products may not be used in any applications where a failure of the product or any consequences of the use thereof can reasonably be expected to result in personal injury.

Edition 2025-06-06 Published by

Infineon Technologies AG 81726 Munich, Germany

© 2025 Infineon Technologies AG. All Rights Reserved.

Do you have a question about this

document?

Email:

erratum@infineon.com

| Document reference 002-15106 Rev. *L | The data contained in this document is exclusively intended for technically trained staff. It is the responsibility of customer’s technical departments to evaluate the suitability of the product for the intended application and the completeness of the product information given in this document with respect to such application. |
| --- | --- |