Description

The AS5403 can measure magnetic fields components in all three dimensions and converts the magnetic field information into absolute position information.

Features

up to 14-bit full scale resolution

33 linearization points to achieve high precision

Absolute linear position sensing up to 40 mm stroke

Insensitive to magnetic stray fields (differential principle)

Programmable output characteristic

150 °C ambient temperature

Build in diagnostics

Overvoltage and reverse polarity protected

Short circuit protection

3D Hall pixels: The AS5403 contains two 3D Hall pixels, spaced 2.5mm apart.

Potential applications

Linear position:

clutch/brake pedal

gearbox sensor

Off-axis

steering angle sensor

gearbox shift link

Product validation

Table of contents

Features	1

Potential applications	1

Product validation	1

Description	1

Table of contents	2

1	Heading 1	3

1.1	Heading 2	3

1.1.1	Heading 3	3

1.1.1.1	Heading 4	3

References	4

Glossary	5

Revision history	6

Disclaimer	7

Preliminary Datasheet

General Description

The AS5403 can measure magnetic fields components in all three dimensions and converts the magnetic field information into absolute position information.

Only a simple 2-pole magnet is required as the magnetic field source.

Using two 3D-Hall cells allows absolute (single pixel) as well as differential (double pixel) 3D magnetic field measurement.

The differential measurement makes the AS5403 ideal for use in rough automotive position sensing applications that include not only dust, dirt or moisture but also unwanted magnetic stray fields.

All the signal conditioning, including compensation of temperature effects, magnet-to-chip gap changes as well as linearization of the output is included in the IC.

The absolute position information of the magnet is directly accessible over a SPI interface and a programmable PWM or analog output. The build in diagnostic functions makes the AS5403 suitable for safety critical applications.

The AS5403 is available in a 14-pin TSSOP package and is qualified according AEC-Q100 for an ambient temperature range from -40°C to +150°C.

It operates at a supply voltage of 5V +/-10%.

The programming of the AS5403 is done over the single wire UART interface.

The AS5403 is overvoltage protected up to 18 V on the supply and output pins. In addition the supply pins are reverse polarity protected up to -18 V.

# Applications

Linear position:

- clutch/brake pedal

- gearbox sensor

Off-axis

- steering angle sensor

- gearbox shift link

AS5403A/B/D/E

3D Hall position sensor for linear and off-axis applications

# Key Features

- up to 14-bit full scale resolution

- 33 linearization points to achieve high precision

- Absolute linear position sensing up to 40 mm stroke

- Insensitive to magnetic stray fields (differential principle)

- Programmable output characteristic

- 150 °C ambient temperature

- Build in diagnostics

- Overvoltage and reverse polarity protected

- Short circuit protection

Figure 1: typical set-up of AS5403A/D with magnet (linear)

Figure 2: typical set-up of AS5403B with magnet (off-axis)

Figure 3: Block diagram of AS5403A/B/D/E

3D Hall pixels: 	The AS5403 contains two 3D Hall pixels, spaced 2.5mm apart.

MUX:	The multiplexer pre-selects depending on the AS5403 variant and chosen mode two/four magnetic components.

ADC:	The Sigma-Delta ADC samples the Hall sensors signals selected by the MUX. The sampling of the sensors is done sequentially.

Signal conditioning: 	This block includes offset and temperature compensation as well as amplitude matching

Bi/Bj:	Preparation of the input signal for the ATAN calculation. Inversion and offset adjustment functions.

ATAN:	Angle calculation.

Linearization: 	A 33-point linearization of the ATAN output. In addition output settings for gain an clamping.

Temperature:	An on-chip temperature sensor is available. It can be read over the SPI interface. This sensor is also used for signal conditioning

PWM interface: 	The linearized measurement data is available over a single pin in the form of a pulse width modulated (PWM) signal.

SPI interface: 	A bi-directional SPI interface allows communication with the chip, including reading measurement data, E²PROM contents or writing configuration data.

E²PROM: 		The on-chip E²PROM contains the configuration data of the chip.

Diagnostics:	Monitor functions on different blocks to check the correctness of the internal signals.

# Pin Configuration

X indicates the axis of lateral position measurement; z axis is perpendicular to the package surface

Figure 4: AS5403 pin configuration, TSSOP-14 package (top view)

## Pin Description

Table 1: Pin description TSSOP14

| Pin | Symbol | Type | Description |
| --- | --- | --- | --- |
| 1 | TEST | DI_PD | Test pin connect to GND on PCB |
| 2 | SCS | DI_PD | SPI chip select (active high), connect to GND if not used in application |
| 3 | SCLK | DI_PD | SPI clock, connect to GND if not used in application |
| 4 | SDI | DI_PD | SPI input data line, connect to GND if not used in application |
| 5 | SDO | DO | SPI output data line, leave open if not used in application |
| 6 | TEST | DO | leave open on PCB |
| 7 | NC |  | Not connected, Set to GND in application |
| 8 | NC |  | Not connected, Set to GND in application |
| 9 | NC |  | Not connected, Set to GND in application |
| 10 | VDD | S | Supply Voltage 5 V |
| 11 | VDD3 | AIO | Regulator output |
| 12 | GND | S | Ground |
| 13 | OUT | AIO | Analog/PWM Output, programming option over output. |
| 14 | DSW | AIO | Programmable digital switch output |

PIN Types:	S		...	supply pad

AIO		...	analog I/O

DI_PD		…	digital input with internal pull down

DO		...	digital output – push-pull

# Electrical Characteristics

## Absolute Maximum Ratings

Stresses beyond those listed under “Absolute Maximum Ratings“ may cause permanent damage to the device. These are stress ratings only. Functional operation of the device at these or any other conditions beyond those indicated under “Operating Conditions” is not implied. Exposure to absolute maximum rating conditions for extended periods may affect device reliability.

Table 3	Absolute maximum ratings

| Parameter | Symbol | Min | Max | Unit | Comments |
| --- | --- | --- | --- | --- | --- |
| DC supply voltage at pin VDD | VDD | -18 | 18 | V |  |
| Voltage at pin OUT and DSW | OUT_OV | -0.3 | 18 | V |  |
| DC voltage at VDD3 pin | VREG | -0.3 | 5 | V |  |
| Input current (latchup immunity) | Iscr | -100 | 100 | mA | AEC-Q100-004 |
| Electrostatic discharge | ESD |  | ± 2 | kV | AEC-Q100-002 |
| EEPROM endurance cycles | EEPcyc |  | 100 | cycles | A part of EEPROM is reserved for factory settings. This part is pre-programmed and locked by ams AG. The customer area of EEPROM can be programmed up to 100 times at Tamb=27deg. EEPROM is intended to be programmed at 0h only in the customer production line and shall not be reprogrammed during operation in the field. |
| Storage temperature | Tstrg | -55 | 150 | °C | Min – 67°F ; Max +257°F |
| Body temperature | TBody |  | 260 | °C | The reflow peak soldering temperature (body temperature) specified is in accordance with IPC/JEDEC J-STD-020 “Moisture/Reflow Sensitivity Classification for Non-Hermetic Solid State Surface Mount Devices”. The lead finish for Pb-free leaded packages is matte tin (100%Sn). |
| Humidity non-condensing | N | 5 | 85 | % |  |
| Moisture Sensitive Level | MSL | 3 |  |  | Represents a maximum floor life time of 168h |

## Operating Conditions

Table 4	Electrical characteristics

Operating conditions: operating temperature = -40 to +150°C, VDD = 4.5 - 5.5V unless otherwise noted.

| Parameter | Symbol | Min | Typ | Max | Unit | Note |
| --- | --- | --- | --- | --- | --- | --- |
| Operating temperature | Tambient | -40 |  | 150 | °C |  |
| Supply current | IDD |  | 20 | 25 | mA |  |
| Positive supply voltage | VDD | 4.5 | 5 | 5.5 | V |  |
| Power up time | TSTUP |  |  | 10 | ms |  |

## Magnetic Sensor Conditions

Table 5	Magnetic characteristics

Operating conditions: operating temperature = -40 to +150°C, VDD = 4.5 - 5.5V unless otherwise noted.

| Parameter | Symbol | Min | Typ | Max | Unit | Note |
| --- | --- | --- | --- | --- | --- | --- |
| Magnetic Range X,Y,Z | BIR | ±51) |  | ±45 | mT | AS5403A (Bx,Bz) |
|  |  | ±51) |  | ±100 | mT | AS5403D (Bx,Bz) |
|  |  | ±51) |  | ±45 | mT | AS5403B (Bx,By) |
|  |  | ±51) |  | ±100 | mT | AS5403E (Bx,By) |
| Sensitivity Matching Bx,Bz | Mxz |  | 3 |  | % |  |
| Sensitivity Matching Bx,By | Mxy |  | 3 |  | % |  |
| Offset drift Bx | ODx | -1 |  | 1 | %FSR2) |  |
| Offset drift Bz | ODz | -0.5 |  | 0.5 | %FSR2) |  |

Note:	1)  minimum condition is valid if both input components are above 5 mT

2) 45 mT AS5403A and AS5403B, 100 mT AS5403D

## DC/AC characteristics for digital pads

Table 6 DC/AC characteristics

| Parameter | Symbol | Min | Typ | Max | Unit | Note |
| --- | --- | --- | --- | --- | --- | --- |
| High level input voltage | VIH | 70 |  |  | %VDD3 |  |
| Low level input voltage | VIL |  |  | 30 | %VDD3 |  |
| Pull-down input current | IPD | 30 |  | 100 | µA |  |
| Pull-up input current | IPU | -100 |  | -30 | µA |  |
| Input leakage current PD | ILPD | -5 |  | 5 | µA |  |
| Input leakage current PU | ILPU | -5 |  | 5 | µA |  |
| High level output voltage | VOH | VDD3-0.5 |  |  | V | IL=-4mA |
| Low level output voltage | VOL |  |  | 0.4 | V | IL=4mA |
| Capacitive load | CL |  |  | 100 | pF |  |

## Output Driver Parameters

Table 7 Output Driver Characteristics

| Parameter | Symbol | Min |  | Typ | Max |  | Unit | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GENERAL |  |  |  |  |  |  |  |  |
| Short circuit output current (LSD) | ILIMITLSD | 5 |  | 10 | 20 |  | mA | VOUT=+18V |
| Short circuit output current (HSD) | ILIMITHSD | -20 |  | -10 | -5 |  | mA | VOUT=0V |
| Short circuit detection time | TSCDET | 20 |  |  | 600 |  | µs | Output stage turned off |
| Short circuit recovery time | TSCREC | 2 |  |  | 20 |  | ms | Output stage turned on |
| Output leakage | ILEAK | -20 |  |  | 20 |  | µA | VOUT=5V; VDD=5V |
| Output voltage broken GND with pull-up | BGNDPU | 96 |  |  | 100 |  | %VDD | Guaranteed by design |
| Output voltage broken GND with pull-down | BGNDPD | 0 |  |  | 4 |  | %VDD |  |
| Output voltage broken VDD with pull-up | BVDDPU | 96 |  |  | 100 |  | %VDD |  |
| Output voltage broken VDD with pull-down | BVDDPD | 0 |  |  | 4 |  | %VDD |  |
| ANALOG |  |  |  |  |  |  |  |  |
| Output level high | OUTVOH | 96 |  |  |  |  | % VDD | IOUT=-3mA |
| Output level low | OUTVOL |  |  |  |  | 4 | % VDD | IOUT=3mA |
| Output Integral non linearity | OUTINL |  |  |  |  | 10 | LSB | Between 4% and 96% of VDD |
| Output Differential non linearity | OUTDNL | -10 |  |  |  | 10 | LSB | Between 4% and 96% of VDD |
| Output Offset | OUTOFF | -25 |  |  |  | 25 | mV | Best fit line offset; evaluated between 4% and 96% of VDD |
| Update rate of the Output | OUTUD |  | 1000 |  |  |  | µs |  |
| Output step response (rising) | OUTSTPR |  |  |  |  | 150 | µs | From step on DAC input to 90% of VDD on the OUT pin; RPUOUT=4.7K; CLOUT=1nF; VDD=5V |
| Output step response (falling) | OUTSTPF |  |  |  |  | 150 | µs | From step on DAC input to 10% of VDD on the OUT pin; RPUOUT=4.7K; CLOUT=1nF; VDD=5V |
| Output Voltage Temperature drift | OUTDRIFT | -0.5 |  |  |  | 0.5 | % | of value at mid code |
| Output ratiometricity error | OUTRATE | -1.5 |  |  |  | 1.5 | % VDD | Between 4% and 96% of VDD (1) |
| Noise | OUTNOISE |  |  |  |  | 25 | mVpp | 1KHz…30kHz; at 2048 LSB level, lab characterization only |
| PWM |  |  |  |  |  |  |  |  |
| PWM frequency | PWMF |  | 1000 |  |  |  | Hz |  |
| PWM Sync time | TSYNCH |  | 0.1*1/PWMF |  |  |  |  |  |
| Output voltage high | PWMVOH | 4.6 |  |  |  |  |  | VDD=5V; IOUT=-5mA |
| Output voltage low | PWMVOL | 0 |  |  |  | 0.4 | V | VDD=5V, IOUT=5mA |
| PWM slew rate (rising edge) | PWMSRR | 1 | 2 |  |  | 4 | V / µs | Between 25% and 75% RPUOUT=10K; CLOUT=1nF VDD=5V (2) |
| PWM slew rate (falling edge) | PWMSRF | 1 | 2 |  |  | 4 | V / µs | Between 75% and 25% RPUOUT=10K; CLOUT=1nF VDD=5V (2) |
| PROGRAMMING |  |  |  |  |  |  |  |  |
| High Level Input Voltage at OUT | OUTVIH | 70 |  |  |  |  | % VDD | VDD=5V |
| Low Level Input Voltage at OUT | OUTVIL |  |  |  |  | 30 | % VDD | VDD=5V |
| UART baud rate | BRATE | 2.4 |  |  |  | 9.6 | KHz | VDD=5V. Error < 1% |

## SPI Timing

Table 8 SPI Timing

| Parameter | Symbol | Min | Typ | Max | Unit | Note |
| --- | --- | --- | --- | --- | --- | --- |
| SCLK period | TSCLK | 125 |  |  | ns |  |
| SCLK high phase | TSCLKH | 62.5 |  |  | ns |  |
| SCLK low phase | TSCLKL | 62.5 |  |  | ns |  |
| SCS lead time | TLEAD | 50 |  |  | ns |  |
| SCS lag time | TLAG | 50 |  |  | ns |  |
| SCS low phase | TSCSL | 2500 |  |  | ns |  |
| SDI input setup time | TSUPI | 50 |  |  | ns |  |
| SDI input hold time | THLDI | 50 |  |  | ns |  |
| SDO output valid time | TVALID |  |  | 50 | ns | CL = 200pF |
| SDO output hold time | THLDO | 0 |  |  | ns | CL = 200pF |
| SDO output access time | TACC |  |  | 50 | ns | CL = 200pF |
| SDO output disable time | TDIS |  |  | 25 | ns | CL = 200pF |
| SDO output rise time | TRISE |  |  | 15 | ns | CL = 200pF |
| SDO output fall time | TFALL |  |  | 15 | ns | CL = 200pF |

1) Receiving mode

2) Transmitting mode

Figure 5 SPI timing diagram

## UART Timing

Table 9 UART Timing

| Parameter | Symbol | Min | Typ | Max | Unit | Note |
| --- | --- | --- | --- | --- | --- | --- |
| UART baud rate | TSCLK | 2.4 |  | 9.6 | kHz |  |

# Functional Description

The AS5403 is manufactured in a CMOS process and uses lateral and vertical Hall sensor technology for sensing the magnetic field distribution in all 3D directions.

The integrated 3D-Hall pixels are placed with a pixel pitch of 2.5 mm and deliver a voltage representation of the magnetic field at the surface of the IC.

Through Sigma-Delta Analog / Digital Conversion and Digital Signal-Processing (DSP) algorithms, the AS5403 provides accurate high-resolution absolute angular position information. For this purpose a Coordinate Rotation Digital Computer (CORDIC) calculates the angle and the magnitude.

## Signal Processing Path Front

The AS5403 can be configured in different operation modes. These are absolute (ABS), average (AVG) and differential (DIFF). The internal calculation scene is changed automatically with the selected mode.

Figure 6: Signal Processing path AS5403A and AS5403D (Bx and Bz)

Figure 7: Signal Processing path AS5403B and AS5403E (Bx and By)

Note: 	yellow 	→functional block

blue	→readable register

green	→write/readable EEPROM parameter

The individual component from 3D Hall pixel 0 and 3D Hall pixel 1 are measured sequentially. Refer to Figure 3. This sequence is pre-defined and depends also on the mode selection (absolute, average or differential). The digital values after conversation are stored in the measurement value registers. MV1….MV4. These values represent the raw data. The pre-calculation leads to internal result values of Bi’ and Bj’. The final Bi and Bj values are available after further manipulation possibilities like offset or gain manipulation. In addition is an exchange of Bi and Bj possible over the swap function. The calculated angle and magnitude can be read out too.

## Internal Calculation Formulas

Table 10 Formulas for Calculation

| Absolute Mode (ABS) | Average Mode (AVG) | Differential Mode (DIFF) |
| --- | --- | --- |
| Bj'=MV1 Bi'=MV3 Bj =Gain Bj*(Bj'+Offset Bj) Bi =Gain Bi*(Bi'+Offset Bi) ATAN2-Bi;Bj | Bj'=(MV1+ MV2)/2 Bi'=(MV3+MV4)/2 Bj =Gain Bj*(Bj'+Offset Bj) Bi =Gain Bi*(Bi'+Offset Bi) ATAN2-Bi;Bj | Bj'=-(MV1-MV2)/2 Bi'=(MV3-MV4)/2 Bj =Gain Bi*(Bi'+Offset Bi) Bi =Gain Bj*(Bj'+Offset Bj) ATAN2-Bi;Bj |

## Signal Processing Path Backend

Figure 8: Signal processing path backend

Note: 	yellow 	→functional block

blue	→readable register

green	→write/readable EEPROM parameter

ATAN2 -BjBi

A pre-scale function can be used to extend the angle range. This is in particular needed in sector applications (off-axis) or small linear strokes. This function optimizes the usage of the linearization function in the following step. The linearization takes maximum 33 supporting point. These points are equally spread over the angular range. The post-processing function is able to manipulate the output characteristic in gain, offset and clamping. The digital output switch function uses a comparator. The switching value and hysteresis can be defined.

Finally the configuration of the output defines the operation of the output drivers.

# Operation

The AS5403 operates at 5V ±10%, using one internal Low-Dropout (LDO) voltage regulator. For operation, the 5V supply is connected to pin VDD. While VDD3 (LDO output) must be buffered by 1µF capacitor, the VDD requires a 1µF capacitor. All capacitors (low ESR ceramic) are supposed to be placed close to the supply pins (see Figure 9).

The VDD3 output is intended for internal use only. It must not be loaded with an external load.

Figure 9: Connections for 5V supply voltages

Note: The pin VDD3 must always be buffered by a capacitor. It must not be left floating, as this may cause instable internal supply voltages which may lead to larger output jitter of the measured angle.

The supply pin is over voltage protected up to 18 V. In addition the device has a reverse polarity protection.

## External Components

Table 11 external components in the system

| Parameter | Symbol | Min | Typ | Max | Unit | Note |
| --- | --- | --- | --- | --- | --- | --- |
| VDD Buffer Capacitance | CVDD | 0.8 | 1 | 1.2 | uF |  |
| ESR of VDD capacitance | ESRCBVDD |  |  | 0.3 |  |  |
| VDD3 Buffer capacitor | CVDD3 | 0.8 | 1 | 1.2 | uF |  |
| ESR of VDD3 capacitance | ESRREG3 |  |  | 0.3 |  |  |
| OUT Load Capacitance | COUT | 0 |  | 33 | nF |  |
| OUT Pull-Up Resistance | RPUOUT | 4.7 |  | 10 | K |  |
| OUT Pull-Down Resistance | RPDOUT | 4.7 |  | 10 | K |  |
| DSW Load Capacitance | CDSW | 0 |  | 33 | nF |  |
| DSW Pull-Up Resistance | RPUDSW | 4.7 |  | 10 | K |  |
| DSW Pull-Down Resistance | RPDDSW | 4.7 |  | 10 | K |  |

# Built In Safety

Table 12 Diagnostic functions of the AS5403

| Monitoring | Error Type | Source | Note |
| --- | --- | --- | --- |
| VDD undervoltage | Hardware-Error(1) | Power management | Recoverable |
| VDD overvoltage | Hardware-Error | Power management | Recoverable |
| VREG undervoltage | Hardware-Error | Power management | Recoverable |
| Oscillator failure | Hardware-Error | CLK management | Not recoverable |
| Loss of GND | Hardware-Error | Output driver |  |
| Loss of VDD | Hardware-Error | Output driver |  |
| Output short circuit | Hardware-Error | Output driver | Recoverable after 16 PWM period |
| PWM/Digital switch readout failure | Hardware-Error | Output driver | Recoverable after 16 PWM period |
| Signature failure | Hardware-Error | EEPROM | Not recoverable |
| Linearization Overflow | Algorithm Error | Digital DSP |  |
| Histogram Warning | Algorithm Error | Digital DSP |  |
| Range Warning | Algorithm Error | Digital DSP |  |
| Sensitivity Correction Overflow | Algorithm Error | Digital DSP |  |
| Normalization Overflow | Algorithm Error | Digital DSP |  |
| Magnet Lost | Algorithm Error | Digital DSP |  |
| Self Monitoring Error | Hardware-Error | Digital DSP | Not recoverable |
| PWM synchronization Error | Hardware-Error | PWM Engine | Not recoverable |

- Hardware-Error  Outputs in HZ

- Algorithm Error   PWM at 5%DC and /PWM at 95%DC depending on Diag_High, digital switches in HZ, FBs for analog output (00 or FF depending on Diag_High)

The residual undetected failure fraction based on the build in diagnostic results in a residual FIT rate of 1.5 FIT. (based on 85 °C, 60% UCL, 0.7 eV)

# Output drivers

AS5403 has two output stages, with different characteristics. The output driver on pin OUT can be programmed as analog output or low side driver PWM (/PWM), and includes a receiver for the bidirectional communication used to configure the device at module level. The driver on pin DSW is a low side only and can be configured as /PWM (PWM) or digital switch but doesn’t have the receiver and in communication behaves as programmed in the EEPROM. Possible configurations for OUT selectable from EEPROM bits are:

Table 13  Possible configurations OUT pin

| Mode | OUT CFG <2> | OUT CFG <1> | OUT CFG <0> | Note |
| --- | --- | --- | --- | --- |
| Analog Output Mode | 0 | 0 | 0 | Push-Pull analog output driver external pull up or pull down required |
| PWM push/pull | 0 | 1 | 0 | Pulse width modulated output with push/pull driver external pull up required |
| PWM open drain | 0 | 1 | 1 | Pulse width modulated output with low side driver external pull up required |
| PWMn push/pull | 1 | 0 | 0 | Inverted pulse width modulated output with push/pull driver external pull up required |
| PWMn push/pull | 1 | 0 | 1 | Inverted pulse width modulated output with low side driver external pull up required |
| PWM open drain reduced slew rate | 1 | 1 | 0 | Pulse width modulated output with low side driver external pull up required reduced falling edge |

Note: All other not specified combinations are reserved and not allowed. To avoid floating situation in diagnostic case a pull up resistor is recommended also in push/pull mode.

## PWM Output

When the PWM mode is selected the measured position is proportional to the duty cycle. A range of 10% to 90% of the PWM period is used to carry the information and the remaining ranges are used as a fail signal (a 5% PWM means Algorithm Error, no change over a whole period means a Hard-error). The 80% of the PWM period that contains the information is divided in 1024 or 4096 segments depending on the selected PWM frequency. The behavior of PWM and /PWM is shown in the following pictures:

Figure 10 PWM output signal

Figure 11 PWM/ output signal

At the end of the power up phase, after an under voltage recovery and after the transition from sleep to normal mode a synch pulse with duration TSYNCH precedes the PWM wave as shown in the following picture. Only after the pulse the digital switches leave the HZ state.

Figure 12 Sync Pulse at PWM after power up

## Digital Switch

The AS5403 provides a digital output switch function. This signal can be defined by settings in the EEPROM. The switching point DSW SW POINT <11:0> are compared to the calculated linearized sensor position. The polarity is selectable and the hysteresis is configurable. Possible configurations for DSW selectable from EEPROM bits are:

Table 14  Possible configurations DSW pin

| Mode | DSW CFG <2> | DSW CFG <1> | DSW CFG <0> | Note |
| --- | --- | --- | --- | --- |
| Digital switch push/pull | 0 | 0 | 0 | Push-Pull digital output driver external pull up required |
| Digital switch open source | 0 | 0 | 1 | High side digital driver external pull down required |
| Digital switch open drain | 0 | 1 | 0 | Low side digital driver external pull up required |
| PWMn push/pull | 0 | 1 | 1 | Inverted pulse width modulated output with push/pull driver external pull up required |
| PWMn open drain | 1 | 0 | 0 | Inverted pulse width modulated output with low side driver external pull up required |
| PWM push/pull | 1 | 0 | 1 | Pulse width modulated output with low side driver external pull up required |
| PWM open drain | 1 | 1 | 0 | Pulse width modulated output with low side driver external pull up required |

Note: All other not specified combinations are reserved and not allowed.

DSW_POL= 1:

Figure 13 DSW characteristic with positive polarity

DSW_POL=0:

Figure 14 DSW characteristic with negative polarity

# 4wire Serial Peripheral Interface (SPI)

AS5403 is equipped with a 4wire serial peripheral interface (SPI) to access the EEPROM memory and the read /write registers. SCS input pin (active high) selects the device for serial transfers. Register data is shifted in from the external master on the SDI pin or shifted out from the device on the SDO pin on each subsequent SCLK, in both the cases MSB first. Data are captured on the rising edge and shifted on the falling edge of SCLK for receiving command and transmitting command. An even parity bit is used to check the consistency of the frame.

SPI protocol is built by frames; each frame is composed by 4 bytes and it is detected only when SCS pin is high.

If a frame contains a number of bits different from the expected the command is not executed.

Valid commands for the SPI interface are the following:

Table 15 Commands SPI

| CMD Name | CMD Value | Note | AS5403 Communication Mode |
| --- | --- | --- | --- |
| Write | 0 | Write data in the memory area | SLAVE |
| Read | 1 | Read data from the memory area | SLAVE/MASTER |

## Write (0)

The first byte of the write command is composed by the command identifier (CMD) the even parity bit (PAR) and the MSBs of the address AD<10:8>.

The second byte of the command is the remaining part of the address AD<7:0>, the third and the fourth byte contains the data word we want to write (D<15:0>) on address AD<10:0>. The device forces the SDO pin low.

With this command it is possible to access the EEPROM locations and the read/write registers area.

Figure 15 SPI Write

## Read (1)

The read command is composed by 2 frames, in the first one the external master sends the command, the even parity and the address to be read on the SDO line (the last 16 bits are ignored). The device forces the SDO pin low.

Figure 16 SPI Read

After the switching time (TSCSL) the device drives the SDO line and when the SCS pin goes high starts to send out the answer. The first byte of the second frame is composed by the command the device is executing, the even parity and the MSBs of the address required in the previous frame (AD<10:8>). The second byte of the command is the remaining part of the address AD<7:0>, the third and the fourth bytes are the data word at the required address (AD<10:0>). With this command it is possible to access the EEPROM locations and the read/write registers area.

# Programming the AS5403

A standard half duplex UART protocol is used to exchange data with the device in the communication mode.

## UART interface for programming

The AS5403 uses a standard UART interface with two bytes for address and two bytes for the data content. The read or write mode is selected in the first byte. An even parity for every byte is included. The timing (baud-rate) is selected by the AS5403 over an initial command from the master. The baud rate register can be read and overwritten. The keep synchronization it AS5403 synchronizes art every Start bit. This happens during a standard write access 3 times.

A time out function detects not complete commands and resets the AS5403 UART after the timeout period.

Table 16 Valid Commands

| CMD Name | Description | AS5403 Communication Mode |
| --- | --- | --- |
| Write | Write data in the EEPROM/ SFR memory | SLAVE |
| Read | Read data from the EEPROM/ SFR memory | SLAVE/MASTER |

In case of Write command the request is followed by the frames containing the data to write.

In case of Read command the communication direction will change and the device will answer with the frames containing the data requested.

### Frame Organisation

The UART frame consists of 1 start bit (low level), 8 data bit, 1 even-parity bit and 1 stop bit (high level). Data are transferred from LSB to MSB

Figure 17 General UART frame

In case of read command the idle phase between the command and the answer is TSW.

Table 17 Bit assignment in frame

| Parameter | Symbol | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| Start bit | START |  | 1 |  | TBIT |
| Data bit | Dx |  | 1 |  | TBIT |
| Parity bit | PAR |  | 1 |  | TBIT |
| Stop bit | STOP | 1 |  |  | TBIT |
| Slave/Master Switch Time | TSW |  | 7 |  | TBIT |

Each communication starts with the reception of a request from the external controller. The request consists of 3 frames: one synchronization byte and 1 word for the command.

The synchronization frame contains the data 0x55 and allows the UART to measure the external controller baud-rate.

Figure 18 Synchronization frame 0x55hex

The 2nd and 3rd frames contain the command Read/ Write (1 bit) and the address (7+7 bits):

Figure 19 2nd and 3rd frame addressing

### Write Command

Synchronization frame:

Write command frame low address:

Write command frame high address:

Data L frame (LSBs of the data to write on address AD<13:0>):

Data H frame (MSBs data to write on address AD<13:0>):

Figure 20 Write Command Frames

### Read Command

Synchronization frame:

Read command frame low address:

Read command frame high address:

Data L frame (LSBs of the data read at the address AD<13:0>):

Data H frame (MSBs of the data read at the address AD<13:0>):

Figure 21 Read Command Frames

### BAUDRATE Automatic Detection

The UART includes a built-in baud-rate monitor that uses the synchronization frame to detect the external controller baud-rate. This baud-rate is used after the synchronization byte to decode the following frame and to transmit the answer and it is stored in the BAUDREG register.

### Programming Procedure

The EEPROM programming is possible over the SPI or UART interface. A page write/read mode with 64 pages 8 words each is implemented.

Table 18 Page mode address

| Address | Page |
| --- | --- |
| 0x000 - 0x007 | 1 |
| 0x008 - 0x00F | 2 |
| : | : |
| 0x1F8 - 0x1FF | 64 |

Page Write mode procedure:

1. Write 0x00 on address 0x2FF (START DSP = 0)

2. Write the EEPROM words (SPI/UART) inside a page (min 1 max 8 words)

3. Write 0x0003 on address 0x2FF (CFG_EPP=11)

4. Wait for 10 ms

5. Repeat from 2. for further programming.

It is important that the write access to the selected EEPROM page are consecutive: a write command in a different page or a read command before writing CFG_EEP will delete the data. If the factory lock bit is set a write access in the factory reserved area are ignored.

After programming it is recommended to read back the EEPROM content and to download again the EEPROM to avoid misalignment with the mirror registers. This can be done with a power up or writing 0x0004 on address 0x2FF (EE_DWNL SFR).

The programming procedure is not allowed in case Customer Lock word is 0x55AA after EEPROM download (mask_fuse=1).

### Lock procedure and signature calculation

The AS5403 contains a signature diagnostic function for the EEPROM. This signature is calculated in the AS5403 during power up and is compared to the calculated signature directly after the end of line calibration by the customer. A deviation leads to an error indication in the diagnostic flag or into the failure band mode at the output. The signature check is enabled in case of a locked device by the user.

# Device Configuration

AS5403 is equipped with a 1kx8 EEPROM memory to store the factory settings and the customer configuration data. The device can be configured using the UART or 4wire Serial Peripheral Interface (SPI).

## EEPROM Memory Map

Note: Write 0x0000 hex to register address 0x2FF (START DSP = 0) before a read or write access to EEPROM.

Table 19	EEPROM Memory Map

| ADDRESS (HEX) | EEPROM Location Name |
| --- | --- |
| 0x00B | Sequencer Control |
| 0x00F | Magnet Lost threshold and hysteresis |
| 0x019 | Offset Bj |
| 0x01A | Gain Bj |
| 0x01E | Gain Bi |
| 0x01F | Offset Bi |
| 0x05F | Linearization Table entry 16 |
| 0x060 | Linearization Table entry 0 |
| 0x061 | Linearization Table entry 1 |
| 0x062 | Linearization Table entry 2 |
| 0x063 | Linearization Table entry 3 |
| 0x064 | Linearization Table entry 4 |
| 0x065 | Linearization Table entry 5 |
| 0x066 | Linearization Table entry 6 |
| 0x067 | Linearization Table entry 7 |
| 0x068 | Linearization Table entry 8 |
| 0x069 | Linearization Table entry 9 |
| 0x06A | Linearization Table entry 10 |
| 0x06B | Linearization Table entry 11 |
| 0x06C | Linearization Table entry 12 |
| 0x06D | Linearization Table entry 13 |
| 0x06E | Linearization Table entry 14 |
| 0x06F | Linearization Table entry 15 |
| 0x070 | Linearization Table entry -16 |
| 0x071 | Linearization Table entry -15 |
| 0x072 | Linearization Table entry -14 |
| 0x073 | Linearization Table entry -13 |
| 0x074 | Linearization Table entry -12 |
| 0x075 | Linearization Table entry -11 |
| 0x076 | Linearization Table entry -10 |
| 0x077 | Linearization Table entry -9 |
| 0x078 | Linearization Table entry -8 |
| 0x079 | Linearization Table entry -7 |
| 0x07A | Linearization Table entry -6 |
| 0x07B | Linearization Table entry -5 |
| 0x07C | Linearization Table entry -4 |
| 0x07D | Linearization Table entry -3 |
| 0x07E | Linearization Table entry -2 |
| 0x07F | Linearization Table entry -1 |
| 0x1CF | Pre-Scale Linearization |
| 0x1D0 | Post-processing Offset |
| 0x1D1 | Post-processing Gain |
| 0x1D2 | Clamp Low 1 |
| 0x1D3 | Clamp High 1 |
| 0x1F5 | Clamp High 2 (MSBs), Clamp Low 2 |
| 0x1F6 | Clamp High 2 (LSBs) |
| 0x1F7 | Angle Offset |
| 0x1F8 | Customer LOCK |
| 0x1F9 | Output configuration and PWM frequency |
| 0x1FA | DSW driver configuration |
| 0x1FB | DSW settings |
| 0x1FC | Customer ID |
| 0x1FD | Customer ID |
| 0x1FE | Customer ID |
| 0x1FF | EEPROM Signature + Customer ID |

### EEPROM Sequencer Control 0x00B

| Register hex | Access | Bit | Function | Default | Note |
| --- | --- | --- | --- | --- | --- |
| 0x00B | R/W | D15 (MSB) | reserved | 0 |  |
|  |  | D14 |  |  |  |
|  |  | D13 |  |  |  |
|  |  | D12 | ExtRng | 0 | Extended Range (for differential mode): 0 = disabled 1 = enabled |
|  |  | D11 | reserved | 0 |  |
|  |  | D10 | Swap | 0 | Swap Bi and Bj before angle calculation |
|  |  | D9 | reserved | 0 |  |
|  |  | D8 | reserved | 0 |  |
|  |  | D7 | Table Select <1> | 0 | These bits allow the selection of 3 different operating modes, stored in 3 individual sequencer tables. Don’t use 11 --> no operation |
|  |  | D6 | Table Select <0> | 1 |  |
|  |  | D5 | reserved | 0 |  |
|  |  | D4 | DIFF | 0 | Differential mode: 0 = absolute measurement of one pixel 1 = differential measurement of both pixels |
|  |  | D3 | reserved | 0 |  |
|  |  | D2 | SeqEn | 1 | 1 = Sequencer enabled 0 = Sequencer disabled |
|  |  | D1 | AVG | 1 | Average mode: 0 = absolute measurement of one pixel, 1 = average measurement of both pixels |
|  |  | D0 (LSB) | reserved | 0 |  |

Table 20 Possible Table Selection AS5403A and AS5403D

|  | Table 0 Table Select 00 | Table 1 Table Select 01 | Table 2 Table Select 10 |
| --- | --- | --- | --- |
| Sensor arrangement | X/Z | X/Z | X/Z |
| EEPROM 0x00B Settings | Absolut Mode Pixel1 1 kHz Sample Rate | Differential/Average Mode 1 kHz Sample Rate | Differential/Average Mode 1 kHz Sample Rate |
| Magnet orientation |  |  |  |

Table 21 Possible Table Selection AS5403B and AS5403E

|  | Table 0 Table Select 00 | Table 1 Table Select 01 | Table 2 Table Select 10 |
| --- | --- | --- | --- |
| Sensor arrangement | X/Y | X/Y | X/Y |
| EEPROM 0x00B Settings | Differential/Average Mode 1 kHz Sampling Rate | Absolute Mode 1 kHz Sampling Rate | Absolute Mode 0.125 kHz Sampling Rate |
| Magnet orientation | off-axis (ring magnet or sector magnet) |  |  |

### EEPROM Magnet Lost 0x00F

| Register hex | Access | Bit | Function | Default | Note |
| --- | --- | --- | --- | --- | --- |
| 0x00F | R/W | D15 (MSB) | reserved | 0 |  |
|  |  | : |  | : |  |
|  |  | D11 |  | 0 |  |
|  |  | D10 | MgnLostHyst<2> | 1 | Magnet lost Hysteresis |
|  |  | : | : | : |  |
|  |  | D8 | MgnLostHyst<0> | 0 |  |
|  |  | D7 | MgnLostLmt<7> | 0 | Magnet lost threshold value compared to register 0x520 |
|  |  | : | : | : |  |
|  |  | D0 (LSB) | MgnLostLmt<0> | 0 |  |

### The magnitude information is compared with the magnet lost threshold value calculated with following formula:

### MagnetLostLimit=MgnLostLmt<7:0>*64

### The hysteresis is calculated using following formula:

Hysteresis=MagnetLostLimit*12MgnLostHyst<2:0>

MgnLostHyst<2:0>=0 disables the hysteresis

### EEPROM Offset Bj 0x019

| Register hex | Access | Bit | Function | Default | Note |
| --- | --- | --- | --- | --- | --- |
| 0x019 | R/W | D15 (MSB) | Offset Bj | 0 | Offset to the Bj value to improve performance Signed integer, range [-32768;32767] |
|  |  | : |  |  |  |
|  |  | D0 (LSB) |  |  |  |

### EEPROM Offset Bi 0x01F

| Register hex | Access | Bit | Function | Default | Note |
| --- | --- | --- | --- | --- | --- |
| 0x01F | R/W | D15 (MSB) | Offset Bi | 0 | Offset to the Bi value to improve performance Signed integer, range [-32768;32767] |
|  |  | : |  |  |  |
|  |  | D0 (LSB) |  |  |  |

### EEPROM Gain Bj 0x01A

| Register hex | Access | Bit | Function | Default | Note |
| --- | --- | --- | --- | --- | --- |
| 0x01A | R/W | D15 (MSB) | Gain Bj | +1 decimal | Gain to the Bj value Signed integer, range [-1;1[ |
|  |  | : |  |  |  |
|  |  | D0 (LSB) |  |  |  |

### EEPROM Gain Bi 0x01E

| Register hex | Access | Bit | Function | Default | Note |
| --- | --- | --- | --- | --- | --- |
| 0x01E | R/W | D15 (MSB) | Gain Bi | +1 decimal | Gain to the Bj value Signed integer, range [-1;1[ |
|  |  | : |  |  |  |
|  |  | D0 (LSB) |  |  |  |

### EEPROM Linearization Table 0x05F to 0x07F

| Register hex | Access | Bit | Content | Default | Note |
| --- | --- | --- | --- | --- | --- |
| 0x05F | R/W | D15 …D0 | Angle linearization table, value 16 | 0000 | Signed integer [-1;1[ |
| 0x060 |  |  | Angle linearization table, value 0 |  |  |
| 0x061 |  |  | Angle linearization table, value 1 |  |  |
| : |  |  | … |  |  |
| 0x06F |  |  | Angle linearization table, value 15 |  |  |
| 0x070 |  |  | Angle linearization table, value -16 |  |  |
| 0x071 |  |  | Angle linearization table, value -15 |  |  |
| : |  |  | … |  |  |
| 0x07E |  |  | Angle linearization table, value -2 |  |  |
| 0x07F |  |  | Angle linearization table, value -1 |  |  |

### EEPROM Pre-Scale Linearization 0x1CF

| Register hex | Access | Bit | Function | Default | Note |
| --- | --- | --- | --- | --- | --- |
| 0x1CF | R/W | D15 (MSB) | Pre-Scale linearization Factor | +1 decimal | Signed integer [-8;8[ |
|  |  | : |  |  |  |
|  |  | D0 (LSB) |  |  |  |

### EEPROM Post-Processing Offset 0x1D0

| Register hex | Access | Bit | Function | Default | Note |
| --- | --- | --- | --- | --- | --- |
| 0x1D0 | R/W | D15 (MSB) | Post processing offset | 0 | Signed integer [-32768;32767[ |
|  |  | : |  |  |  |
|  |  | D0 (LSB) |  |  |  |

### EEPROM Post-Processing Gain 0x1D1

| Register hex | Access | Bit | Function | Default | Note |
| --- | --- | --- | --- | --- | --- |
| 0x1D1 | R/W | D15 (MSB) | Post processing gain | +1 decimal | Signed integer [-4;4[ |
|  |  | : |  |  |  |
|  |  | D0 (LSB) |  |  |  |

### EEPROM Clamp Low 1 0x1D2

| Register hex | Access | Bit | Function | Default | Note |
| --- | --- | --- | --- | --- | --- |
| 0x1D2 | R/W | D15 (MSB) | Clamp Low 1 | -32768 decimal | Signed integer [-32768;32767[ |
|  |  | : |  |  |  |
|  |  | D0 (LSB) |  |  |  |

### EEPROM Clamp High 1 0x1D3

| Register hex | Access | Bit | Function | Default | Note |
| --- | --- | --- | --- | --- | --- |
| 0x1D3 | R/W | D15 (MSB) | Clamp High 1 | 32767 decimal | Signed integer [-32768;32767[ |
|  |  | : |  |  |  |
|  |  | D0 (LSB) |  |  |  |

### EEPROM Clamp Low 2 and Clamp High 2 (LSBs) 0x1F5

| Register hex | Access | Bit | Function | Default | Note |
| --- | --- | --- | --- | --- | --- |
| 0x1F5 | R/W | D15 (MSB) | Clamp High 2 (LSBs) | 0 | Clamp High 2 (LSBs) value for PWM and analog output. DSW not effected. 4 lower LSBs [0;16[ |
|  |  | : |  | : |  |
|  |  | D12 |  | 0 |  |
|  |  | D11 | Clamp Low 2 | 0 | Clamp Low 2 value for PWM and analog output. DSW not effected. 12 bit unsigned value [0;4096[ |
|  |  | : |  | : |  |
|  |  | D0 (LSB) |  | 0 |  |

### EEPROM Clamp High 2 (MSBs) 0x1F6

| Register hex | Access | Bit | Function | Default | Note |
| --- | --- | --- | --- | --- | --- |
| 0x1F6 | R/W | D15 (MSB) | reserved | 0 |  |
|  |  | : |  | : |  |
|  |  | D8 |  | 0 |  |
|  |  | D7 | Clamp High 2 (MSBs) | 0 | Clamp High 2 (MSBs) value for PWM and analog output. DSW not effected. 8 higher MSBs [0;256[ |
|  |  | : |  | : |  |
|  |  | D0 (LSB) |  | 0 |  |

### EEPROM Angle Offset 0x1F7

| Register hex | Access | Bit | Function | Default | Note |
| --- | --- | --- | --- | --- | --- |
| 0x1F7 | R/W | D15 (MSB) | Angle Offset | 0 | Signed integer [-32768;32767[ |
|  |  | : |  |  |  |
|  |  | D0 (LSB) |  |  |  |

### EEPROM Customer Lock 0x1F8

| Register hex | Access | Bit | Function | Default | Note |
| --- | --- | --- | --- | --- | --- |
| 0x1F8 | R/W | D15 (MSB) | Customer Lock | 0 | EEPROM lock key region. When key 0x55AA is programmed --> no more write access to complete EEPROM |
|  |  | : |  |  |  |
|  |  | D0 (LSB) |  |  |  |

### EEPROM OUT pin configuration and PWM frequency 0x1F9

| Register hex | Access | Bit | Function | Default | Note |
| --- | --- | --- | --- | --- | --- |
| 0x1F9 | R/W | D15 (MSB) | OUT FALL <1> | 0 | Threshold for the fall time check. 00 = no check 01 = 24 – 28 µs 10 = 56 – 60 µs 11 = 120 – 124 µs |
|  |  | D14 | OUT FALL <0> | 0 |  |
|  |  | D13 | OUT RISE <1> | 0 | Threshold for the rise time check. 00 = no check 01 = 24 – 28 µs 10 = 56 – 60 µs 11 = 120 – 124 µs |
|  |  | D12 | OUT RISE <0> | 0 |  |
|  |  | D11 | reserved | 0 |  |
|  |  | D10 | OUT CFG <2> | 0 | Output driver configuration OUT pin. |
|  |  | : | : | : |  |
|  |  | D8 | OUT CFG <0> | 0 |  |
|  |  | D7 | reserved | 0 |  |
|  |  | : |  | : |  |
|  |  | D3 |  | 0 |  |
|  |  | D2 | DIAG_HIGH | 0 | Failure band selection internal errors 0=Failure band low 1= Failure band high |
|  |  | D1 | PWMF <1> | 0 | PWM frequency selection 00 = 1 kHz. Don’t use other settings except AS5403B/E 11=0.125kHz in case of Table Select = 10 |
|  |  | D0 (LSB) | PWMF <0> | 0 |  |

### EEPROM DSW pin configuration 0x1FA

| Register hex | Access | Bit | Function | Default | Note |
| --- | --- | --- | --- | --- | --- |
| 0x1FA | R/W | D15 (MSB) | DSW SW POINT <7:0> | 0 | Low byte of the switching point |
|  |  | : |  | : |  |
|  |  | D8 |  | 0 |  |
|  |  | D7 | DSW FALL <1> | 0 | Threshold for the fall time check. 00 = no check 01 = 24 – 28 µs 10 = 56 – 60 µs 11 = 120 – 124 µs |
|  |  | D6 | DSW FALL <0> | 0 |  |
|  |  | D5 | DSW RISE <1> | 0 | Threshold for the rise time check. 00 = no check 01 = 24 – 28 µs 10 = 56 – 60 µs 11 = 120 – 124 µs |
|  |  | D4 | DSW RISE <0> | 0 |  |
|  |  | D3 | reserved | 0 |  |
|  |  | D2 | DSW CFG <2> | 0 | Output driver configuration DSW pin. See Table 13 |
|  |  | D1 | : | 0 |  |
|  |  | D0 (LSB) | DSW CFG <0> | 0 |  |

### EEPROM DSW Settings 0x1FB

| Register hex | Access | Bit | Function | Default | Note |
| --- | --- | --- | --- | --- | --- |
| 0x1FB | R/W | D15 (MSB) | reserved | 0 |  |
|  |  | D14 | reserved | 0 |  |
|  |  | : |  | 0 |  |
|  |  | D10 |  | 0 |  |
|  |  | D9 | reserved | 0 |  |
|  |  | D8 | reserved | 0 |  |
|  |  | D7 | DSW_POL | 0 | Polarity: 0 = low level 1 = high level |
|  |  | D6 | DSW HYST <2> | 0 | DSW Hysteresis: 000 = 4 LSB; 001 = 8 LSB; 010 = 16 LSB; 011 = 32 LSB; 100 = 48 LSB; 101 = 64 LSB; 110 = 96 LSB; 111 =128 LSB; LSB are on 12 bits |
|  |  | : | : | 0 |  |
|  |  | D4 | DSW HYST <0> | 0 |  |
|  |  | D3 | DSW SW POINT <11> | 0 | Upper 4 bits of the switching point |
|  |  | : | : | : |  |
|  |  | D0 (LSB) | DSW SW POINT <8> | 0 |  |

### EEPROM Customer ID 0x1FC

| Register hex | Access | Bit | Function | Default | Note |
| --- | --- | --- | --- | --- | --- |
| 0x1FC | R/W | D15 (MSB) | CUST ID | 0 | Customer ID number 7 to 0 |
|  |  | : |  | : |  |
|  |  | D8 |  | 0 |  |
|  |  | D7 | CUST ID | 0 | Extended Byte |
|  |  | : |  | : |  |
|  |  | D0 (LSB) |  | 0 |  |

### EEPROM Customer ID 0x1FD

| Register hex | Access | Bit | Function | Default | Note |
| --- | --- | --- | --- | --- | --- |
| 0x1FD | R/W | D15 (MSB) | CUST ID | 0 | Customer ID number 23 to 8 |
|  |  | : |  | : |  |
|  |  | D0 (LSB) |  | 0 |  |

### EEPROM Customer ID 0x1FE

| Register hex | Access | Bit | Function | Default | Note |
| --- | --- | --- | --- | --- | --- |
| 0x1FE | R/W | D15 (MSB) | CUST ID | 0 | Customer ID number 39 to 24 |
|  |  | : |  | : |  |
|  |  | D0 (LSB) |  | 0 |  |

### EEPROM Customer ID and signature 0x1FF

| Register hex | Access | Bit | Function | Default | Note |
| --- | --- | --- | --- | --- | --- |
| 0x1FF | R/W | D15 (MSB) | EESIGN | 0 | EEPROM Signature |
|  |  | : |  | : |  |
|  |  | D8 |  | 0 |  |
|  |  | D7 | CUST ID | 0 | Customer ID number 47 to 40 |
|  |  | : |  | : |  |
|  |  | D0 (LSB) |  | 0 |  |

## Register Memory Map

Table 22 Registers Map

Note: Write 0x0100 hex to register address 0x2FF (START DSP = 1) before a read of following registers.

| ADDRESS (HEX) | Register Location Name |
| --- | --- |
| 0x2FF | Control DSP |
| 0x300 | Error Register 1 |
| 0x301 | Position 2 |
| 0x302 | Error Register 2 |
| 0x510 | Temperature |
| 0x511 | Measurement Value 1 (MV 1) |
| 0x512 | Measurement Value 2 (MV 2) |
| 0x513 | Measurement Value 3 (MV 3) |
| 0x514 | Measurement Value 4 (MV 4) |
| 0x520 | Magnitude |
| 0x521 | Angle (not linearized) |
| 0x522 | Angle linearized |
| 0x547 | Position 1 |
| 0x548 | Bi component |
| 0x549 | Bj component |
| 0x54B | Bi’ component |
| 0x54C | Bj’ component |
| 0x54D | Bk (unused) |

### Control DSP 0x2FF

| Register | Access | Bit | Function | Note |
| --- | --- | --- | --- | --- |
| 0x2FF | R/W | D15 (MSB) | reserved |  |
|  |  | : |  |  |
|  |  | D12 |  |  |
|  |  | D11 | PASS2FUNC | Activation for functional mode of the output. |
|  |  | D10 | FREEZE | When CAL_EN=1 it enables the 3D Hall Core for 1 only measurement (automatically cleared at the end of the measurement). Used For SPI calibration in communication mode. When CAL_EN=0 it freezes the read SFR content |
|  |  | D9 | CAL_EN | Enable the calibration procedure. Used in communication mode. |
|  |  | D8 | START_DSP | Start the 3D Hall Core. Used in communication mode to start the DSP measurements. |
|  |  | D7 | reserved |  |
|  |  | : |  |  |
|  |  | D4 |  |  |
|  |  | D3 | EE_RESET | Force EEPROM reset. Used in communication mode. |
|  |  | D2 | EE_DWNL | Force EEPROM download. Used in communication mode. |
|  |  | D1 | CFG_EPP <1> | EEPROM programming mode: 00: no EEPROM permanent write; 11: permanent write of the current EEPROM page (bits CFG_EPP are automatically cleared); 01: permanent write after each EEPROM write command. |
|  |  | D0 (LSB) | CFG_EPP <0> |  |

### Error Register 1 0x300

| Register | Access | Bit | Function | Note |
| --- | --- | --- | --- | --- |
| 0x300 | R | D15 (MSB) | reserved |  |
|  |  | D14 | MgnLost | Magnet Lost: it indicates that Magnitude is below the configured threshold (magnet is too far away) (EEPROM 0x00F) |
|  |  | D13 | DiPaSeMo | Digital Part Self-Monitoring fail: it indicates that the digital part self-monitoring detected an error. |
|  |  | D12 | NrmOvfl | Normalisation Overflow: it indicates the multiplier overflow during normalisation. |
|  |  | D11 | SensOvfl | Sensitivity Correction Overflow: it indicates the Multiplier overflow during sensitivity correction over temperature |
|  |  | D10 | RngWarn | Range Warning: it indicates that the ADC input signal exceeds the input range. |
|  |  | D9 | HistWarn | Histogram Warning: it indicates an improper distribution of ADC output values inside the possible range. From the possible 16 digital values at the modulator output a ‘histogram’ is calculated. |
|  |  | D8 | CalcError | Calculation Error: it indicates that an overflow in post-processing calculations (additions in dynamic linearization direct path) occurred. |
|  |  | D7 | reserved |  |
|  |  | D6 | SLOvfl | Linearization Overflow: it indicates that the multiplication in front of the linearization saturated. |
|  |  | D5 | GainSat | Gain Multiplication Saturation: it indicates that the result of the gain multiplication saturated. |
|  |  | D4 | ClampStatus 1 | Clamp Status 1: it indicates that the post-processing used the clamping values to limit the output value. |
|  |  | D3 | ClampStatus 2 | Clamp Status 2: it indicates that the unsigned post-processing used the clamping values to limit the PWM/Analog value. |
|  |  | D2 | reserved |  |
|  |  | : |  |  |
|  |  | D0 (LSB) |  |  |

### Error Register 2 0x302

| Register | Access | Bit | Function | Note |
| --- | --- | --- | --- | --- |
| 0x302 | R | D15 (MSB) | reserved |  |
|  |  | : |  |  |
|  |  | D11 |  |  |
|  |  | D10 | WD_ERR | Watchdog error. |
|  |  | D9 | DWL_BIST_ER | EEPROM signature error is generated during the download into SFR mirror |
|  |  | D8 | TST_ERR | Signature error on TEST SFR (they should be at 0 during functional mode |
|  |  | D7 | SYNCH_ERR | Synchronization error between 3D Hall Core data rate and PWM frequency |
|  |  | D6 | reserved |  |
|  |  | D5 | RD_BACK_ERR2 | Read back error on DSW pin |
|  |  | D4 | RD_BACK_ERR1 | Read back error on OUT pin |
|  |  | D3 | reserved |  |
|  |  | D2 | DSW_SHORT | Short circuit error on DSW pin |
|  |  | D1 | PWM_SHORT | Short circuit error on OUT pin |
|  |  | D0 (LSB) | BIST_ERR | EEPROM signature error |

### Register Temperature 0x510

| Register hex | Access | Bit | Function | Note |
| --- | --- | --- | --- | --- |
| 0x510 | R | D15 (MSB) | Temperature | Temperature [°C] = (17325 + value [LSB] )/ 315 |
|  |  | : |  |  |
|  |  | D0 (LSB) |  |  |

### Register Measurement Value 1..4 MV1…MV4 0x511, 0x512, 0x513, 0x514,

| Register hex | Access | Bit | Function | Note |
| --- | --- | --- | --- | --- |
| 0x511 0x512 0x513 0x514 | R | D15 (MSB) | MV1 MV2 MV3 MV4 | Measurement values of 3D Hall Pixels |
|  |  | : |  |  |
|  |  | D0 (LSB) |  |  |

### Register Magnitude 0x520

| Register hex | Access | Bit | Function | Note |
| --- | --- | --- | --- | --- |
| 0x520 | R | D15 (MSB) | Magnitude | Magnitude information after ATAN calculation |
|  |  | : |  |  |
|  |  | D0 (LSB) |  |  |

### Register Angle not linearized 0x521

| Register hex | Access | Bit | Function | Note |
| --- | --- | --- | --- | --- |
| 0x521 | R | D15 (MSB) | Angle | Angle information after Cordic calculation |
|  |  | : |  |  |
|  |  | D0 (LSB) |  |  |

### Register Angle linearized 0x522

| Register hex | Access | Bit | Function | Note |
| --- | --- | --- | --- | --- |
| 0x522 | R | D15 (MSB) | Angle linearized | Angle information after linearization. 16 bit linearized output |
|  |  | : |  |  |
|  |  | D0 (LSB) |  |  |

### Register Position 1 0x547

| Register hex | Access | Bit | Function | Note |
| --- | --- | --- | --- | --- |
| 0x547 | R | D15 (MSB) | Position 1 | Position 1 value signed output |
|  |  | : |  |  |
|  |  | D0 (LSB) |  |  |

### Register Position 2 0x301

| Register hex | Access | Bit | Function | Note |
| --- | --- | --- | --- | --- |
| 0x301 | R | D15 (MSB) | Position 2 | Position 2 value unsigned output |
|  |  | : |  |  |
|  |  | D0 (LSB) |  |  |

### Register Bi Component 0x548

| Register hex | Access | Bit | Function | Note |
| --- | --- | --- | --- | --- |
| 0x548 | R | D15 (MSB) | Bi | Magnetic field component Bi. Content dependant on sequencer control settings and table selection |
|  |  | : |  |  |
|  |  | D0 (LSB) |  |  |

### Register Bj Component 0x549

| Register hex | Access | Bit | Function | Note |
| --- | --- | --- | --- | --- |
| 0x549 | R | D15 (MSB) | Bj | Magnetic field component Bj. Content dependant on sequencer control settings and table selection |
|  |  | : |  |  |
|  |  | D0 (LSB) |  |  |

### Register Bi’ Component 0x54B

| Register hex | Access | Bit | Function | Note |
| --- | --- | --- | --- | --- |
| 0x54B | R | D15 (MSB) | Bi‘ | Magnetic field component Bi’. Content dependant on sequencer control settings and table selection |
|  |  | : |  |  |
|  |  | D0 (LSB) |  |  |

### Register Bj’ Component 0x54C

| Register hex | Access | Bit | Function | Note |
| --- | --- | --- | --- | --- |
| 0x54C | R | D15 (MSB) | Bj‘ | Magnetic field component Bj’. Content dependant on sequencer control settings and table selection |
|  |  | : |  |  |
|  |  | D0 (LSB) |  |  |

## Temperature Sensor

The on chip temperature sensor can be used to gain rough information about the ambient temperature. The SPI interface is mandatory when this information shall be used.

Table 23	Temperature Sensor

| Parameter | Conditions | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| Temperature Signal | @ 0°C |  | -17325 |  | LSB |
| Temperature Sensor resolution |  |  | 315 |  | LSB/°K |

## Protections

When the voltage applied to the VDD pin falls below the undervoltage lower threshold (VDDUVTL) for longer than the TVDDUVDET time the device stops the clock of the digital part, it resets the EEPROM signature calculation and the 3D Hall core and the output drivers are turned off to reduce the power consumption. When the voltage applied to the VDD pin exceeds the VDD undervoltage upper threshold (VDDUVTH) for longer than the TVDDUVREC time the clock, the signature calculation and the 3D Hall core are restarted and the output drivers are turned on.

Table 24	Single Wire Bit Timing

| Parameter | Symbol | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| VDD Undervoltage Upper Threshold | VDDUVTH | 3.7 | 4.1 | 4.5 | V |
| VDD Undervoltage Lower Threshold | VDDUVTL | 3.3 | 3.7 | 4.1 | V |
| VDD Undervoltage Detection Time | TVDDUVDET | 10 |  | 250 | us |
| VDD Undervoltage Recovery Time | TVDDUVREC | 10 |  | 250 | us |

## Sensor Placement

Two 3D Hall pixels each with an X-/Y-/Z-Sensor are arranged in a line on the X Axis parallel to the chip edge, 2.5mm distant from each other.

3D Hall pixel positions relative to chip centre are:

Pixel 1: -1250 mm

Pixel 0: 1250 mm

Figure 22: 3D Hall pixel arrangement

# Package Drawings and Markings

## 14-Lead Thin Shrink Small Outline Package TSSOP-14

Figure 23: TSSOP-14 Package Dimensions and Marking

# Ordering Information

The devices are available as standard products, shown in Table 24.

| Model | Description | Delivery Form | Package |
| --- | --- | --- | --- |
| AS5403A-HTST | Absolute Linear Position Sensor with ±45 mT magnetic input range Bx/Bz | Tape & Reel | TSSOP 14 |
| AS5403D-HTST | Absolute Linear Position Sensor with ±100 mT magnetic input range Bx/Bz | Tape & Reel | TSSOP 14 |
| AS5403B-HTST | Off-Axis Position Sensor with ±45 mT magnetic input range Bx/By | Tape & Reel | TSSOP 14 |
| AS5403E-HTST | Off-Axis Position Sensor with ±100 mT magnetic input range Bx/By | Tape & Reel | TSSOP 14 |

Table 25: Ordering Information

Note: All products are RoHS compliant and ams green.

Buy our products or get free samples online at ICdirect: http://www.ams.com/ICdirect

Technical Support is available at http://www.ams.com/Technical-Support

For further information and requests, please contact us mailto: sales@ams.com

or find your local distributor at http://www.ams.com/distributor

# Revision History

| Revision | Description | Change date |
| --- | --- | --- |
| 0.6 | 2 magnetic input ranges, Specification of possible configuration Tables. Table select configurations added. SPI protocol change. | 21.12.2012 |
| 0.7 | Update of EEPROM addresses and register addresses. | 20.02.2012 |
| 0.8 | Include 45 and 100 mT version AS5403A and AS5403D. Update of Table 16 | 03.06.2012 |
| 0.9 | Pass2Func bit added, EEPORM endurance cycles added, 3D pixel placement added, Sensor matching and offset values added. | 12.06.2013 |
| 0.10 | Include Version AS5403B, Details of signal processing added. Block diagram update. | 17.07.2013 |
| 0.11 | Additional functions included. Gain change Bi and Bj. Angle offset and swap function. | 10.02.2014 |
| 0.12 | Formula Correction Table 9. Value change 12.1.5 and 12.1.5 to [-1;1[ | 07.03.2014 |
| 0.13 | Change of formula Table 9 differential mode. Formula change 12.2.4 Temperature Sensor | 30.04.2014 |
| 0.14 | Default pin configuration 7, 8 and 9 changed. Formula magnet lost diagnostics added. TSynch time added. | 02.06.2014 |

Table 26: revision history

# Copyrights

Copyright © 1997-2014, ams AG, Tobelbaderstrasse 30, 8141 Unterpremstaetten, Austria-Europe. Trademarks Registered ®. All rights reserved. The material herein may not be reproduced, adapted, merged, translated, stored, or used without the prior written consent of the copyright owner.

All products and companies mentioned are trademarks or registered trademarks of their respective companies.

# Disclaimer

Devices sold by ams AG are covered by the warranty and patent indemnification provisions appearing in its Term of Sale. ams AG makes no warranty, express, statutory, implied, or by description regarding the information set forth herein or regarding the freedom of the described devices from patent infringement. ams AG reserves the right to change specifications and prices at any time and without notice. Therefore, prior to designing this product into a system, it is necessary to check with ams AG for current information. This product is intended for use in normal commercial applications. Applications requiring extended temperature range, unusual environmental requirements, or high reliability applications, such as military, medical life-support or life-sustaining equipment are specifically not recommended without additional processing by ams AG for each application. For shipments of less than 100 parts the manufacturing flow might show deviations from the standard production flow, such as test flow or test location.

The information furnished here by ams AG is believed to be correct and accurate. However, ams AG shall not be liable to recipient or any third party for any damages, including but not limited to personal injury, property damage, loss of profits, loss of use, interruption of business or indirect, special, incidental or consequential damages, of any kind, in connection with or arising out of the furnishing, performance or use of the technical data herein. No obligation or liability to recipient or any third party shall arise or flow out of ams AG rendering of technical or other services.

Contact Information

Headquarters

ams AG

Tobelbaderstrasse 30

A-8141 Unterpremstaetten, Austria

Tel : +43 (0) 3136 500 0

Fax : +43 (0) 3136 525 01

For Sales Offices, Distributors and Representatives, please visit:

http://www.ams.com/contact

References

Glossary

AA
active authentication (AA)

AAUI
application activation user interface (AAUI)

AC
alternating current (AC)

AC
access control (AC)

ACLB
advanced contactless bridge (ACLB)
The ACLB interface is used to connect the security controller acting in the role of contactless IC (CIC) to an external analog contactless frontend (ACF), both together forming (part of) a “Boosted NFC” system. The data exchange via the ACLB interface is based on symbol level.

BDT
buffered data transfer (BDT)
The controller is in a low-power state while a communication peripheral transfers data to or from an I/O buffer.

BER
basic encoding rules (BER)

BIST
built-in self-test (BIST)
A mechanism that permits the chip to test itself.

Etc.

Revision history

| Document revision | Date | Description of changes |
| --- | --- | --- |

Disclaimer