AS5116

Magnetic Position Sensor with Analog Outputs

General Description

The AS5116 is a contactless magnetic position sensor for accurate angular measurement over a full mechanical turn of 360°. Based on the Hall sensor technology, this device has a robust architecture that measures the orthogonal component of the flux density (Bz), over a full-turn rotation. To measure the angle, only a simple two-pole magnet rotating over the center of the package is required.

With two decades of magnetic sensor system know-how, we have broad experience in selecting the most suitable magnets. ams experts recommend the following magnet suppliers.

Key Features

Figure 1:
Added Value of Using AS5116

| Benefits | Features |
| --- | --- |
| Highest reliability and durability | Contactless angle measurement |
| Accurate angle measurement | Low output noise |
| Low system costs – no shielding required | Low inherent INL |
| Enabler for safety critical applications | Magnetic stray field immunity overachieves ISO 11452-8 |
| High precision analog output | Developed according to ISO26262 |
| Small form factor | Fully differential buffered sine and cosine output signals |
| Fully automotive-qualified | AEC-Q100, grade 0 |

Applications

| Rotor angle sensing in the field of | Electric pumps |
| --- | --- |
| Automotive applications | Actuators in transmission systems |
| Electric power steering systems | Starter/Generator systems |

Block Diagram

Figure 2:
Functional Blocks of AS5116